Reg.No. ____________

End Semester Examination – Nov/Dec – 2017

| Code : | 17EE3014 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | PHOTOVOLTAIC SYSTEMS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Draw the equivalent circuit of a solar cell and discuss the electrical characteristics of the solar cell. Also derive the equations for the same. | CO4 | 15 |
|  | b. | Determine the Zenith angle, solar altitude angle and solar azimuth angle on 21st Jan of 11 A.M solar time at a location 48 N. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | With the help of neat diagrams explain about the mismatches happened in the solar cell/ module because of series and parallel connection of solar cell/module. | CO4 | 10 |
|  | b. | Estimate the monthly average daily global solar radiation on the horizontal surface at Chennai(13.0827° N, 80.2707°E) during the month of April if the average sunshine hours per day is 10.2. Assume values for a=0.27 and b=0.50. | CO1 | 10 |
| 3. | a. | With the help of a neat sketch explain the construction and working principle of Crystalline Silicon Solar cells. | CO4 | 10 |
|  | b. | Compare the different types of solar cells with respect to the efficiency, materials and Voltage. | CO4 | 10 |
| (OR) |  |  |  |  |
| 4. |  | Design a solar PV System wherein the total load consists of CFL,TV, fan, refrigerator and a computer . The system should allow the use of loads in non-sunshine hours. The operating hours and power rating of these loads are given in the table below. S.NO; LOAD; WATTS; H/DAY; NUMBER. 1; CFL; 18; 6; 3. 2; Fan; 70; 4; 2. 3; TV(27”); 250; 8; 1. 4; Refrigerator; 150; 2; 1. 5; Computer; 250; 1; 1. | CO6 | 20 |
| 5. | a. | With the help of neat diagrams explain about the power. conditioning circuits used in PV system and derive an expression for voltage gain for the types of DC-DC Converters. | CO1 | 10 |
|  | b. | Describe the principle of working of Lead acid batteries and state its advantages. | CO1 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Briefly explain about the different MPPT methods used in solar PV system. State its advantages and limitations for each method? | CO1 | 10 |
|  | b. | Design the sizing and energy balance of a grid connected Photovoltaic Systems. | CO3 | 10 |
| 7. | a. | Draw the circuit diagram of the three phase inverter circuit. Also draw its necessary waveforms and explain the operation of the circuit. | CO1 | 10 |
|  | b. | Discuss about the rural electrification in India. | CO5 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Design a PV water pumping system, which is required to draw 20,000 litres of water every day from a depth of 20mt. | CO2 | 10 |
|  | b. | Briefly explain about the building integrated photovoltaic plants. | CO5 | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | With the help of a schematic diagram describe about the construction, design and working model of solar powered car. | CO6 | 20 |

ALL THE BEST