Reg.No. ____________

End Semester Examination – Nov/Dec– 2017

| Code : | 17ME3001 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | FINITE ELEMENT METHODS IN ENGINEERING | Max. marks : | 100 |

Answer ALL questions (5 x 20 = 100 MARKS)

| Q. No | Sub. Div | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Solve the given simultaneous equation by Gaussian elimination method. x1 – 2x2 + 6x3 = 0 2x1 + 2x2 + 3x3 = 3 – x1 + 3x2 = 2 | CO1 | 10 |
|  | b. | Find the dominant Eigen values and the corresponding eigenvector of the matrix. H = 2−10−12−10−12 | CO1 | 10 |
|  |  | (OR) |  |  |
| 2. | a. | Elucidate the following system of equation using Choleski decomposition method: x +3y +8z =4 x + 4y +3z = -2 x + 3y + 4z = 1 | CO1 | 10 |
|  | b. | List the advantages, disadvantages and the applications of FEM. | CO1 | 5 |
|  | c. | What are the difference between boundary value problem and initial value problem? | CO1 | 5 |
| 3. | a. | Derive the shape function for one dimensional quadratic element. | CO2 | 10 |
|  | b. | For the beam and loading shown in figure. Determine the slope at 2 and 3 and the vertical deflection at the mid-point of the distributed load. | CO2 | 10 |
|  |  | (OR) |  |  |
| 4. | a. | Develop the shape function for one dimensional cubic element. | CO2 | 10 |
|  | b. | Consider a bar as shown in fig. Young’s Modulus E = 2 x 105 N/mm2. A1 = 2 cm2, A2 = 1 cm2 and force of 100 N. Calculate the nodal displacement, stresses in each element and reaction forces. | CO2 | 10 |
| 5 |  | A pump pumping fluid at Q = 6500 m3/hr is located at co-ordinates (5, 2) in the element as shown in the figure. Find the amount of Q allotted to each node. All nodal co-ordinates are in mm. Assume unit thickness t = 1 mm. | CO4 | 20 |
|  |  | (OR) |  |  |
| 6. |  | A Composite wall consists of three materials, as shown in Fig. The inner temperature is To=6000C. Convection heat transfer takes place on the outer surface of the wall with T∞=300C and h=45 Wm2 0K. Determine the temperature distribution in the wall. k1=8.5 Wm 0K, k2=0.25 Wm 0K, k3=0.08 Wm 0K, h=45 Wm2 K, Tϖ=300 C | CO3 | 20 |
| 7. | a. | Determine the value of  at a point A in a bilinear rectangular element I = 115, j=85, k=76, m=105. | CO5 | 6 |
|  | b. | Derive the shape functions for a rectangular element in global coordinator system. | CO5 | 14 |
|  |  | (OR) |  |  |
| 8. | a. | Descend the shape function for a 4 noded rectangular elements. Use natural co-ordinate system. | CO5 | 6 |
|  | b. | Develop the shape function for nine noded rectangular element in (ξ,η) coordinate system. | CO5 | 14 |
| 9. |  | Compulsory: Consider a uniform cross-section bar, as shown in figure. Of length L made up of material whose Young’s modulus and density is given by E and ρ. Estimate the natural frequencies of axial vibration of the bar using both consistent and lumped mass matrices. | CO6 | 20 |

ALL THE BEST