Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	AIRCRAFT STRUCTURES								      Time : 3 hours

Subject Code:	09AE202										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	________ is the line of cross section about which product moment of inertia zero.

2.	In a thin walled structure of length ‘L’, the relation between the shear flow and angle of twist subjected to torque is ________.

3.	At high altitude of flying, the pressurized cabin subjected to________ stresses.

4.	In lumped cross sections, the booms assumed to carry the ________stresses.

5.	A ________ is one which is supported on more than two

5kN

5m

A

B

C

R1

R2

60

30

supports.

6.	The stiffness factor equation for the fixed hinged case of beam is ________.

7.	The strain energy stored by the body within the elastic limit is called ________ .

8.	Castigliano’s theorem applies only to the structures having ________ temperature variation.

9.	The effective length value of the column of fixed-hinged condition is ________.

10.	What is the other name for shear strain energy theory?

PART – B  (5 x 3 = 15 MARKS)

11.	Define Shear center and locate the same for ‘L’ and ‘T’ sections.

12.	Write down the three purpose of frames (rings) structure of the fuselage

13.	What are the assumptions made in finding the frame forces?

14.	State Castigliano’s theorem.

15.	What is beam column?  Give examples.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the direct stress distribution due to bending of an unsymmetrical cross section and also the neutral axis position.

(OR)

17.	A thin walled idealized single cell beam is shown in the figure. If the section supports a vertical shear load of 10kN acting in a vertical plane through booms 3and 6, calculate the shear flow around the section. Booms areas are: B1=B8=200mm2, B2=B7=200mm2, B3=B6=200mm2, B4=B5=200mm2.

[P.T.O.]

18.	Determine the shear flow in the web of the tapered beam shown in the figure at a section midway along its length. The web of the beam has a thickness of 2mm and is fully effective in resisting direct stress. The beam tapers symmetrically about its horizontal axis and the cross sectional area of each flange is 400mm2.

(OR)

19.	The fuselage of a light passenger carrying aircraft has the circular cross-section shown below. The cross- sectional area of each stringer is 100mm2 and the vertical distances are to the midline of the section wall at the corresponding stringer position. If the fuselage is subjected to a bending moment of 200 KNm applied in the vertical plane of symmetry at the section. Calculate the direct stress distribution.

20.	An inclined truss loaded as shown in the figure. Determine the forces of the member by any one method.

(OR)

21.	A continuous beam ABCD consists of three span and is loaded as shown in the figure. Find the supported end moments and bending moment diagram using Moment distribution method.

22.	a.	State and prove the Maxwell’s Reciprocal theorem.

b.	Find the Horizontal deflection at B for the circular arc beam shown in figure subjected to load of P.

(OR)

23.	a.	Derive the “Maxwell’s reciprocal theorem”.                                    					  (10)

b.	Write the equation of strain energy caused by bending and shear.           				    (3)

c.	Write the equation of strain energy caused by torsion?                     					    (2)

24.	a.	Explain about “ Euler’s hyperbola ”.

b.	Derive the Rankine’s Hypothesis.

(OR)

25.	A circular shaft of tensile yield strength 260 MPa is subjected to a combined state of loading defined by a bending moment M=15 kN-m and Torque T= 20kN-m. Calculate the diameter of the shaft by applying the following theories.

a.	Maximum shear stress theory.													    (5)

b.	Strain energy theory.															    (5)

c.	Shear strain energy theory.														    (5)