Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	GAS DYNAMICS										      Time : 3 hours

Subject Code:	09AE207										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is compressible fluid flow?

2.	___________ is associated with the flow with Mach number is greater than ‘5’.

3.	What is Rarefaction (expansion) wave?

4.	Fanno curve for a particular flow rate, Mach number is unity where the flow is ___________.

5.	___________ is defined as the ratio of the increase in static pressure across the shock to the upstream pressure.

6.	Fluids with higher values of bulk modulus have higher ___________ .

7.	What is small perturbation potential theory?

8.	Define lift.

9.	What is wind tunnel?

10.	What is the most advantage of optical methods of flow visualization?

PART – B  (5 x 3 = 15 MARKS)

11.	How basic properties vary in nozzle and diffuser along the flow direction?

12.	Write short notes on Rayleigh Flow.

13.	Define Mach Wave and Mach Angle.

14.	How can shock waves be minimized in high speed aerofoil?

15.	What are the shortcomings of continuous supersonic wind tunnel?

PART – C  (5 x 15 = 75 MARKS)

16.	Consider a flow of gas into a converging nozzle with a mass flow rate of 1kg/sec and the entrance area is 0.009 m2 and the exit area is 0.003 m2. The stagnation temperature is 400K and the pressure at point 2 was measured as 5 bar Calculate the net force acting on the nozzle and pressure at point 1.

(OR)

17.	Describe the behavior of flow in a convergent-divergent nozzle when it is operated at:

a.	Design pressure ratio.

b.	Pressure ratio higher than design value.

c.	Pressure ratio lower than design value.

18.	Air flows from a reservoir and enters a uniform pipe with a diameter of 0.05 m and length of   10 m. The air exits to the atmosphere. The following conditions prevail at the exit: P2 = 1bar temperature T2 = 27oC M2 = 0.9. Assume that the average friction factor to be f = 0.004 and that the flow from the reservoir up to the pipe inlet is essentially isentropic. Estimate the total temperature and total pressure in the reservoir under the Fanno flow model.

(OR)

[P.T.O.]

19.	Air enters a pipe with pressure of 3 bar and temperature of 27oC at Mach number of           	M = 0.25. Due internal combustion heat was released and the exit temperature was found to be 	127oC. Calculated the exit Mach number, the exit pressure, the total exit pressure, and heat 	released (transferred) to the air. After what amount of energy the exit temperature will start to 	decrease? Assume Cp = 1.004 kJ/kgoC.

20.	What is supersonic flow? Explain the solutions for supersonic flows.

(OR)

21.	Detail the linearized two dimensional supersonic flow theory.

22.	Explain the following in related with airfoil in high speed flow:

a.	Shock induced separation.

b.	Lift and drag divergence.

(OR)

23.	Detail the following in related with airfoil in high speed flow:

a.	Characteristics swept wings

b.	Camber and aspect ratio of wings

24.	The data of a Mach 2 supersonic wind tunnel is given below:

| Pressure in the test section | : 0.69 bar |
| --- | --- |
| Area of cross section of nozzle throat | : 1000 cm2 |
| Ambient pressure and temperature | : 1.02 bar, 311 K |

The air is taken from atmosphere and compressed continuously in a multistage compressor to the reservoir pressure. The test section of tunnel directly exhausts into the atmosphere. Determine:

a.	Temperature of air in the test section.

b.	Mass flow rate of air.

c.	Cross sectional area of the test section.

d.	Power required to drive the compressor. Assume reversible flow throughout.

(OR)

25.	a.	What is the basic principle employed in the working of optical devices for obtaining a 	density field of a gas stream?													    (5)

b.	Draw a neat sketch of an interferometer. How is used to determine the gas velocity and its 		Mach number?																  (10)