Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	SPACE DYNAMICS								      	     Time : 3 hours

Subject Code:	09AE208										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write down the thrust equation of a rocket motor.

2.	Define the specific impulse of a rocket.

3.	What is  the need for stability and control for rockets.

4.	When a spacecraft passes in front of a planet, its velocity will ____________.

5.	What is aero-braking?

6.	What is a retro-burn?

7.	What is flyby mission?

8.	What is orbiter mission?

9.	What is coasting phase in space launch vehicle structural design.

10.	When the effective exhaust velocity of a vehicle in gravitationless space is 2000 m/sec, the mass ratio necessary to boost the vehicle velocity by 1600 m/sec is___________.

PART – B  (5 x 3 = 15 MARKS)

11.	Discuss the fundamental reasons for staging in rockets.

12.	Define: Stability, Equilibrium and Control

13.	Define the term ‘Ecliptic’. And State Kepler’s second law of planetary motion.

14.	Calculate the perturbations in longitude of the ascending node caused by the Moon for a space vehicle orbiting in a circular orbit of radius 7000 km, an inclination of 45 deg, and with an orbital period of 92 minutes.

15.	The period of revolution of a satellite is 106 minutes. Find the apogee altitude if the perigee altitude is 200 km. (Radius of the Earth = 6378 km, μ = 398600.44 km3/s2)

PART – C  (5 x 15 = 75 MARKS)

16.	A rocket projectile has the following characteristics:

Initial mass = 200kg

Mass after rocket operation = 130 kg

Payload, nonpropulsive structure, etc., = 110 kg

Rocket operation duration = 3.0 sec

Average specific impulse of propellant = 240 sec

Determine the vehicle’s (a) mass ratio, (b) propellant mass fraction, (c) propellant flow rate,

(d) thrust, (e) thrust-to-weight ratio, (f) acceleration of the vehicle, (g) effective exhaust velocity, (h) total impulse, and (i) the impulse to weight ratio.

(OR)

17.	a.	Mention the stage separation techniques for a multistage launch vehicle in atmosphere. 	Describe any two techniques with neat illustration.								(10)

b.	Explain the Staging of rockets.				            		 	 					(5)

[P.T.O.]

18.	a.	Describe the Static Longitudinal stability and control of rockets.					(10)

b.	Discuss the dynamic stability of rockets.										(5)

(OR)

19.  	Distinguish stability and control and also explain in detail the Lateral and directional stability and control of rockets

20.	Discuss the effects of Space Environment on the selection of materials for Spacecraft Systems.

(OR)

21.	a.	Describe the various Coordinate Systems used in Space Applications.			  	(8)

b.	Discuss the Earth’s atmosphere and space environment.			                          	(7)

22.	a.	Explain the method of vibrations of orbital elements and also explain what the general 			perturbations approach is.														(10)

b.	Explain in detail the Cowell’s Method.											(5)

(OR)

23   	Referring to figure, consider an initial direct, circular Earth orbit of radius 9100 km and a final direct, coplanar, elliptical orbit with e = 0.1 and radius of perigee 9000 km. What velocity change is required to make the transfer?

(Earth’s gravitational parameter, µ = 398600.4418 km³/s²)

24.	For a Mars mission, calculate the minimum delta-V required to transfer a spacecraft from a circular parking orbit of 185 km altitude around the earth to a highly elliptical capture orbit around the Mars with a periapsis altitude of 450 km and an apoapsis altitude of 80,000 km. Assume that the planets have coplanar circular orbits around the Sun. Also find the time of flight for that transfer.

(Mean distance from Earth to the Sun is 1 AU which is 149.6 x 106 km, Mean distance from Mars to the Sun is 1.524 AU, Sun’s gravitational parameter = 1.327×1011 km³/s², Earth’s gravitational parameter = 398600.4418 km³/s², Mars’ gravitational parameter = 42828.3 km³/s², Radius of Earth = 6378 km, Radius of Mars = 3397 km).

(OR)

25.	a.	Describe the Launch of Interplanetary Spacecraft.					 			(7)

b.	Discuss in detail about the Fast Interplanetary Trajectories	.						(8)