Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	AIRCRAFT PROPULSION							      	     Time : 3 hours

Subject Code:	09AE209										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List out the classification of gas turbine engines.

2.	What are the types of thrust augmentation in gas turbine engines?

3.	The exit blade angle for a forward curved blade should be __________.

4.	What are the main losses in centrifugal compressor?

5.	Explain the types of combustion chambers.

6.	What is the purpose of dilution air in the combustion chamber?

7.	What is choked nozzle?

8.	Define optimum expansion nozzle.

9.	The performance of a simple gas turbine depends on ___________.

10.	Name the types of turbines used in gas turbine engines.

PART – B  (5 x 3 = 15 MARKS)

11.	Define specific fuel consumption and write its significance.

12.	What are the advantages of centrifugal compressor?

13.	Write down the steps involved in the combustion process.

14.	Define thrust reversing.

15.	Explain the basic components in a gas turbine engines.

PART – C  (5 x 15 = 75 MARKS)

16.	In a turbojet unit with forward facing ram intake, the jet velocity relative to the propelling nozzle at exit is twice the flight velocity. Determine the rate of fuel consumption in kg/s, when developing a thrust of 25000 N under the following conditions.

Ambient pressure and temperature		:	0.7 bar, 1 ºC

Compression total head pressure ratio	:	5:1

Flight speed							:	8000km/h

Calorific value of fuel					:	42000 kJ/kg

Ram efficiency						:	100%

Isentropic efficiency 	of compressor	:	85%

Isentropic efficiency of turbine			:	90%

Isentropic efficiency of nozzle			:	95%

Combustion efficiency				:	98%

Turbine pressure ratio					:	2.23

Assume the mass flow rate of fuel is small compared with the mass flow of air and that the working fluid throughout has the properties of air at low temperature. Neglect the extraneous pressure drop. Assume CPg=CPa=1.005 kJ/kgK.

(OR)

[P.T.O.]

17.	a.	Derive the thrust equation for a simple jet engine. 								(8)

b.	Explain the types of thrust augmentation. 										(7)

18.	Determine the impeller diameters and the width at the impeller exit and the power required to drive the compressor. From the following.

Speed	: 						12,5000 rev/min

Mass flow rate: 					15kg/s

Pressure ratio: 					4:1

Isentropic efficiency: 				75%

Slip factor: 						0.9

Flow coefficient at impeller exit:	 0.3

Hub diameter of the eye:			15

Axial velocity of air at entry to

And exit from impeller:			150 m/s

Stagnation temperature at inlet:		295 k

Stagnation pressure at inlet:		1.0 bar

Assume equal pressure ratio in the impeller and diffuser

(OR)

19.	Air at a temperature of 200 K enters a ten stage axial flow compressor at the rate of 3kg/s. the pressure ratio is 6.5 and the isentropic efficiency is 90%, the compression process being adiabatic. The compressor has symmetric blades. The axial velocity of 110m/s is uniform across stage and the mean blade speed of each stage is 180 m/s.

Determine the direction of the air at entry to and exit from the rotor and the stator blades and also the power given to the air. Assume Cp=1.005 kJ/kg k and specific heat ratio is 1.4.

20.	a.	Explain the combustion process in simple gas turbine combustor.					(8)

b.	Explain the types of combustion chamber. 										(7)

(OR)

21.	a. 	Discuss the significance of the elementary theory and vortex theory.				(10)

b.	 Briefly explain the axial and radial flow turbines.								(5)

22.	a.	Explain the components in afterburner. 											(5)

b.	Explain the process of fuel injection and atomization in afterburner.				(10)

(OR)

23.	a.	Derive Area-Mach number relation for an isentropic flow through a variable area passage.

(9)

b.	Explain the basic operation of a propelling nozzle. 								(6)

24.	Explain the matching procedure of a twin spool engine.

(OR)

25.	Explain the transient behavior of gas turbine.