Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	PRINCIPLES OF CHEMICAL ENGINEERING			      Time : 3 hours

Subject Code:	09BT203										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Volume is measured in m3 and litres in ____________ unit.

2. 	____________ is defined as the ratio of molar mass to velocity.

3. 	____________ operation is used for the separation of the components of a liquid mixture by partial vaporization and condensation.

4. 	____________ is the number that precedes the formula of each component involved in a chemical reaction.

5. 	The continuity equation represents the law of conservation of _____________.

6. 	Pressure drop for a fluid flowing in turbulent flow through a pipe is a function of velocity            as _____________.

7. 	Power loss in an orificemeter is ______________ than in a venturimeter.

8. 	____________ may be termed as variable area flow meter.

9. 	Plunger pumps are used for ____________.

10. 	While starting the centrifugal pump, its delivery valve should be kept ____________.

PART – B  (5 x 3 = 15 MARKS)

11. 	15 kg of CO2 is compressed at a temperature of 303 K to a volume of 0.5 m3. Calculate the pressure required for the given duty. Assume ideal gas law is applicable.

12. 	Write the material balance equation for evaporation and leaching.

13. 	Calculate the specific weight and density of 1 litre of a liquid which weighs 7 N.

14. 	What is dense phase and lean phase fluidization?

15. 	Distinguish between centrifugal pump and reciprocating pump.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Cracked gas from a petroleum refinery has the following composition by volume: 	Methane-45%, Ethane-10%, Ethylene-25%, Propane-7%, Propylene-8% and n-butane-5%. 	Determine:	i.	the average molar mass of the gas mixture	ii.	the composition by 	mass 	iii.	specific gravity of the gas mixture.						     			  (10)

b.	A mixture of N2 and CO2 at 298 K and 101.325 k Pa has average molecular weight of 31. 	What is the partial pressure of N2?				                                           			    (5)

(OR)

17.	a.	Water is flowing through a pipe line of 8 cm in diameter. The local velocities at various 	radial positions are given below. Calculate the average velocity, u by:

i. 	Trapezoidal rule 			ii.	Simpson’s one third rule.	   				       	  (10)

| u, cm/s | 3.0 | 2.95 | 2.75 | 1.75 | 1.20 | 0.90 | 0.70 | 0.50 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| r, cm | 0 | 0.5 | 1.0 | 1.5 | 2.0 | 2.5 | 3.0 | 3.5 | 4.0 |

b.	Calculate the density of air containing 21% O2 and 79% N2 by volume at 503 K and 	1519.875 k Pa pressure.						    	 							    (5)

[P.T.O.]

18.	a.	It is required to make 1000 kg mixed acids containing 60% H2SO4, 32% HNO3 and 8% 	water by blending:	i.	the spent acid containing 11.3% HNO3, 44.4% H2SO4 and 	44.3% 	water		ii.	aqueous 90% HNO3 		iii.	aqueous 98% H2SO4. All % are 	by weight. Calculate the quantities of each of the three acids required for blending.	  (10)

b.	2000 kg of wet solids containing 70% solids by weight are fed to a tray dryer where is 	dried by hot air. The product finally obtained is found to contain 1% moisture by weight. 	Calculate:	i.	the kg of water removed from wet solids 	ii.	the kg product 		obtained. 																	    (5)

(OR)

19.	a.	In a textile industry, it is desired to prepare 24% caustic soda solution by weight. Because 	of very high heat dissolution of caustic soda in water, the above solution is prepared in two 	process step. First, caustic soda is dissolved in correct quantity of water in a dissolution 	tank to prepare 50% by weight solution. After dissolution and cooling is complete, this 	solution is taken to a dilution tank where some more water is added for producing 24% by 	weight of caustic soda solution. Assuming no evaporation loss of water in dissolution tank. 	Calculate the weight ratio of water fed to the dissolution tank to bypass water to the 	dilution tank.			                                   										  (10)

b.	In production of SO3, 100 kmole of SO2 and 200 kmole of O are fed to a reactor. The product stream is found to contain 80 kmole SO3. Find the % conversion of oxygen.	    (5)

20.	a.	The efficiency η of a fan depends on density ρ, dynamic viscosity of the fluid µ, angular 		velocity ω, diameter of the rotor D and the discharge Q. Express η in terms of 	dimensionless parameters.										           			  (10)

b.	Discuss the various properties of fluids.				     	      	 					    (5)

(OR)

21.	a.	Explain the principle and operation of different types of pressure measuring device with 	neat diagram.					 					           					 (10)

b.	Discuss the different types of similarities.	 				           				    (5)

22.	a.	Explain the principle and operation of orifice meter with neat diagram.		 		    (8)

b.	An oil of specific gravity 0.8 is flowing through a venturimeter having inner diameter       	20 cm and throat diameter 10 cm. The oil-mercury differential manometer shows a reading 	of 25 cm. Calculate the discharge of oil through the horizontal venturimeter.

Take Cd = 0.98.            														    (7)

(OR)

23.	a.	Explain the principle, operation and application of butterfly valve and ball valve with neat 	diagram.						      				           							  (10)

b.	Write note on particulate fluidization and aggregate fluidization.		           		   (5)

24.	a.	Explain the principle, operation and applications of positive reciprocating pump with neat 	diagram.                     									           					  (10)

b.	Discuss the characteristics curve of centrifugal pump. 				 			    (5)

(OR)

25.	a.	Explain the principle and operation of airlift pump with neat diagram.	        		    (8)

b.	Distinguish between fans, blowers and compressors.								    (7)