Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	UNIT OPERATIONS									      Time : 3 hours

Subject Code:	09BT205										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Unit operations involve bringing a ___________change.

2.	Give an example for the best conductor of heat.

3.	Generally, the expansion of liquids and gases when heated gives rise to___________ currents in fluids due to changing densities.

4.	Which type of heat transfer can occur in vacuum?

5.	In a heat exchanger, floating head is provided to ___________ caused by thermal expansion.

6.	Black liquor generated during paper manufacture is concentrated in a _____________.

7.	What is the difference between deionization and filtration?

8.	Centrifuge is a device used in science to separate suspensions by ____________.

9.	Swirling and vortex are prevented using _____________.

10.	____________is a mixture of two or more liquids which are normally immiscible.

PART – B  (5 x 3 = 15 MARKS)

11.	State Fourier’s law of heat conduction with its equation.

12.	What is meant by Nucleate boiling and Film boiling?

13.	Hot water (0.01 m3 /min) enters the tube side of a counter current shell and tube heat exchanger at 80°C and leaves at 50°C. Cold oil (0.05 m3/ min) of density 800 kg/m3 and specific heat of 2 kJ/ kg K enters at 20°C. Calculate the log mean temperature difference in °C.

14.	Define coagulation and flocculation process.

15.	Prove that Reynolds number is a dimensionless number.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Derive the steady state heat conduction equation through an extended surface. 		    (6)

b.	A furnace is constructed with 229 mm thick fire brick, 115 mm of insulation brick and again 229 mm of building brick. The inside temperature is 1223 K and the temperature at the outermost wall is 323K.The thermal conductivities of fire brick, insulations brick and building brick are 6.05, 0.58 and 2.33 W/mk.  Find the heat lost per unit area and temperature at the interfaces.  													    (9)

(OR)

17.	a.	Give an expression for overall heat transfer coefficient in case of cylindrical surfaces with 	combined conduction and convection. 											    (9)

b.	State the relationship between overall heat transfer coefficient and individual heat transfer 	coefficients. 																	    (6)

18.	State the Buckingham’s pi- Theorem. Explain the various parameters used in forced convection and using dimensional analysis obtain an expression for Nusselt number in terms of Reynolds number and Prandtl number.

(OR)

[P.T.O.]

19.	a.	Explain the drop-wise and Film-type condensation. 								    (8)

b.	Write a short note on the concept of black and grey body. 							    (7)

20.	a.	Derive an expression for calculating LMTD for a co-current type of heat exchanger, which 	is exchanging heat from hot fluid to cold fluid. 									  (10)

b.	List out the different methods of feeding system involved in multiple effect evaporation. 																				    (5)

(OR)

21.	a.	Water is flowing at the rate of 1.13 kg/s with heat capacity 4180 kJ/kg K in a 1-2 shell and 	tube heat exchanger and is heated from 45°C to 85°C by an oil having a heat capacity of 	1.95 kJ/kg K at the rate of 2.5 kg/s. The oil enters at 120°C and leaves at 85°C. Calculate 	the area of the exchanger if the overall heat transfer coefficient is 300 W/m2K. 		    (9)

b.	An evaporator operating at atmospheric pressure is fed at the rate of 10000 kg/hr of weak liquor containing 4 % caustic soda. Thick liquor leaving the evaporator contains 25% caustic soda. Find the capacity of the evaporator. 								    (6)

22.	a.	Derive the equations for constant rate filtration and constant pressure filtration. 		    (9)

b.	Write in brief note on centrifugal filters. 										    (6)

(OR)

23.	a.	Briefly discuss the construction and working of rotary drum filters with a neat sketch. (9)

b.	Write a short note on free settling and Hindered settling. 							    (6)

24.	a.	Derive an expression for dimensional analysis of power correlations.				    (9)

b.	List out the equipments for mixing of powder, viscous material and pastes. 			    (6)

(OR)

25.	a.	Discuss in detail about the agitation equipments with a neat diagram. 				    (9)

b.	Elaborate on Agitator scale up. 												    (6)