Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	MASS TRANSFER OPERATIONS						      Time : 3 hours

Subject Code:	09BT206			       				               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give the mathematical equations for Daltons & Amongst law.

2.	Write the equation for Eddy diffusion.

3.	Suggest methods for the separation of two or more components of a liquid solution.

4.	List out the different types of reflux ratio.

5.	Give the various types of packing materials used in an absorption tower.

6.	What are the systems of absorption?

7.	Define adsorbate and adsorbent.

8.	What is the application of ion exchange in Bio technology field?

9.	Define leaching.

10.	What is the separation of medicinal compounds from plants called as?

PART – B  (5 x 3 = 15 MARKS)

11.	State Fick’s law of Diffusion.

12.	Define relative volatility and give its conditions.

13.	Write down the selection criteria for solvent in gas absorption.

14.	Differentiate between the physical and chemical adsorption.

15.	Differentiate between the Leaching and Extraction.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Discuss about molecular diffusion mechanism with a neat sketch.                			(5)

b.	An open circular tank 6.0 meters in diameter, contains benzene at 25o C which is exposed 		to the atmosphere in such  a manner that the liquid is covered with a stagnant air                        		film having an estimated thickness of 5.0mm. The concentration of benzene beyond the 			stagnant film is negligible. The vapour pressure of benzene at 25o C is 100mm Hg. If 			benzene is worth one rupees a litre, what is the value of the loss of benzene from this tank 		in one rupees per day? Molar diffusivity of benzene in air at 25o C and 1.0 atm pressure is 		277.7 cm2/hr. Density of benzene is 0.88gm/ml.									(10)

(OR)

17.	a.	A tube of small diameter was filled with acetone (density = 0.79gm/cc) upto 1.10 cm from

top and maintained at a  20oC in a gentle current of air. After 5 hrs the levels of liquid fell to 2.05cm from the top. Calculate the diffusivity of acetone in air, if the barometric pressure was 750mmHg.VapPressure of acetone at 20o C is 180mm Hg.

(molecular weight of Acetone =58)												(7)

b.	Explain about inter phase mass Transfer theories, Film theory, Penetration theories with 			a neat diagram.			    													(8)

[P.T.O]

18.	a.	Derive the Rayleigh’s   equation with  a neat sketch.								(5)

b.	A mixture of 35 mole% A and 65 mole% B is to be separated in a fractionating column. 			The Concentrations of A in the distillate is 93mole% and 96% of all product A is in the 			distillate. The feed is half  vap and the reflux ratio is to be 4.0. The relative volatility of A 		to B is 2.0.Calculate the number of theoretical plates in the column and locate the feed 			plate.                                            												(10)

(OR)

19.	a.	A continuous  fractionating column is to be  designed to separate 350 gm moles/minutes of 	a binary mixture containing 40 % (weight) of benzene and 60 % (weight) of toluene. The 	top product contains 97 % (weight) of benzene and bottom product contains 98% (by 	weight) of toluene. A reflux ratio of 3.5 moles to 1 mole of product is to be used. The feed 	is liquid entering the column at its boiling point. (molecular weights of benzene and 	toluene are 78, 92).															(10)

i.	Determine the number of ideal plates.

ii.	Calculate the moles of overhead product and bottom product.

iii.	Find out the location of feed plate.

| X | 0.100 | 0.020 | 0.30 | 0.40 | 0.05 | 0.07 | 0.08 | 0.09 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y | 0.185 | 0.36 | 0.50 | 0.61 | 0.70 | 0.84 | 0.90 | 0.95 | 1 |

b.	Describe about the following industrial distillation equipments.

i.	Extractive distillation 		ii.	Azeotropic distillation equipments.         		(5)

20.	a.	Explain the working procedure of absorption column with a neat sketch				(5)

b.	Ammonia – air mixture containing 2 % Ammonia at 25 °C and 1 atmosphere is to be scrubbed with water in a sieve plate column. The water  rate is 32.6 kg / square meter  sec. 98 % Ammonia  is to be recovered. Calculate the number of ideal stages assuming  1.5  times that of min flow rate, isothermal operation. (Constant temperature)    			(10)

| PA mm Hg | 12 | 18.1 | 31.7 | 50 | 69.6 | 10 |
| --- | --- | --- | --- | --- | --- | --- |
| CA gm of NH3/100gm of Water | 2 | 3 | 5 | 7.5 | 10 | 20 |

(OR)

21.	a.	Describe the packed  absorption column with a sketch , packing methods, packing 				materials. 					 												(8)

b.	5000 kg/hr of a S02-air mixture containing 5% by volume SO2 is to be scrubbed with 200000 kg /hr of water in packed tower. The exits concentration of SO2 is reduced to 0.15%. The tower operates at 1 atmosphere. The equilibrium is given by Y=30X	where	 y = moles of sulphur dioxide / moles of air, X = moles of sulphur dioxide/moles of water. Find the exit concentration of water.							              			(7)

22.	a.	Derive the material  balances for  single  stage operation of fluid- solid operation.	(7)

b.	An aqueous solution containing valuable solute is colored by small amounts of an impurity. Decolonization experiments of an aqueous solution yielded the following equilibrium relationship Y= 8.91x105X1.66, Y= color units/kg of solution and X= color units/kg of carbon. 1000 kg of initial solution with color concentration of 9.6 color unit/kg solution is to be with adsorbents. Calculate the percent of original color removed in single stage using 32kg of fresh Adsorbents.										        (8)

(OR)

23.	a.	Explain the principles and equilibria of ion exchange operations.                     		(7)

b.	Discuss about adsorption isotherms 					      	     					(8)

[P.T.O]

24.	Explain with neat sketches, Bollman Extractor and Rotor cell extractor.

(OR)

25.	Write briefly about:

a.	Heap leaching 		b.	Solution Mining			c.	Shanks