Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	CHEMICAL THERMODYNAMICS AND BIO THERMODYNAMICS														      					     Time : 3 hours

Subject Code:	09BT229										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	For any process, the second law requires that the entropy change of the system be:

a.	+ve or 0		b.	–ve or 0		c.	0		d.	no requirement.

2.	Which of the following may be treated as a statement of the second law of thermodynamics:

a.	Heat and work are emery in transit		b.	It is impossible to convert mechanical work to heat with 100% efficiency		c.	Absolute zero of temperature cannot be attained.

d.	Energy of an isolated system is conserved.

3.	Density is an example of ___________ property.

4.	Gibbs free energy is defined as ___________.

5.	The quantity of heat supplied at constant volume condition is equal to change in _________ of the system.

6.	What is the criterion for the reaction equilibrium?

7.	Vant Hoff equation is used to study the relationship between ___________ and ____________.

8.	Liquefaction makes ___________, _________ and __________ gases simple.

9.	What is meant by anabolism?

10.	What is meant by catabolism?

PART – B  (5 x 3 = 15 MARKS)

11.	Show that Cp–Cv = R for an ideal gas.

12.	Explain the importance of partial molar properties with the help of OH-Water system.

13.	What are the three general methods of engineering importance for liquefaction of gases?

14.	Write a note on effect of pressure on equilibrium constant.

15.	Mention the example for anabolic and catabolic process.

PART – C  (5 x 15 = 75 MARKS)

16.	Calculate the change in internal energy, change in enthalpy, work done and the heat supplied in the following process.

a.	An ideal gas is expanded from 5 bar to 4 bar isothermally at 600 K.

b.	An ideal gas contained in a vessel of 0.1 m3 capacity is initially at 1 bar and 298 K. It is heated at constant volume to 400 K (Assume that Cp=30J/mol K).

(OR)

17.	One kmol of CO2 occupies a volume of 0.381 m3 at 313 K. Compare the pressures given by

a.	Ideal gas equation.

b.	Van der Waals equation, b= 4.28 x 10 –5 a = 0.365 Nm4/mol2.

18.	Derive Gibbs-Duhem equation in terms of activity co-efficient for a binary solution.

(OR)

[P.T.O.]

19.	The volume of an aqueous solution of NaCl at 298 K was measured for a series of molalities (moles of solute per kg of solvent) and it was found that the volume varies with molality according to the following expression,

V= 1.003× 10-3 + 0.1662 × 10-4m + 0.176 × 10-5 m1.5 + 0.12 × 10-6 m2

where m is the molality and V is in m3. Calculate the partial molar volumes of the components at m = 0.1 mol/kg.

20.	In the synthesis of NH3, stoichiometric amount of N2 and H2 are sent to a reactor where the following reaction takes place. N2+3H2NH3. The equilibrium constant for the reaction at 675 K may be taken equal to 2x10-4. Determine the percentage conversion of N2 to NH3 at 675 K and 20 bar.

(OR)

21.	Derive the relationship between free energy change and equilibrium constant with the help of following stoichiometry: aA + bB ↔ cC + Dd.

22.	Describe in detail the Linde process for gas-liquefaction with a neat flow chart.

(OR)

23.	Discuss in detail the Rankine cycle with its components and TS diagram.

24.	Assume the experimental measurements for a certain organism have shown that cells can convert two-thirds (wt/wt) of the substrate carbon (glucose) to 	biomass.

Calculate the stoichiometric coefficients for the following biological reaction:

Glucose: C6H12O6 + a O2 + b NH3 → c (C4.4 H7.3 N0.86 O1.2) + d H2O+ e CO2

Also calculate the yield co-efficients YX/S.

(OR)

25.	Production of single cell protein from hexadecane is described by the following reaction:

C16H34+ a O2 + b NH3 → c (CH1.66 N0.20 O0.27) + d CO2 + eH2O

Where CH1.66 N0.20 O0.27 represents biomass. If RQ = 0.43; determine the stoichiometric coefficients.