Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	MECHANICS OF FLUIDS								      Time : 3 hours

Subject Code:	09CE202										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the relationship between dynamic viscosity and kinematic viscosity?

2.	What is the buoyancy?

3.	What is the path line?

4.	What is the stream tube?

5.	What is the use of pitot tube?

6.	Name the equipments used for discharge measurement in open channels.

7.	Name the equation used for calculating frictional losses.

8.	What is the purpose of syphon?

9.	Write the classification of hydraulic models.

10.	What is the dimensional homogeneity?

PART – B  (5 x 3 = 15 MARKS)

11.	Define ‘Meta centric height’.

12.	Write the classification of flows.

13.	Differentiate notches and weirs.

14.	Define ‘water hammer’.

15.	What do you mean by ‘scale effect’?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Name the equipments used to measure the pressures.								    (5)

b.	A rectangular plate 3m long and 2m wide is immersed vertically in water in such that its     3 m side is parallel to the water surface and is 2m below it. Find:

i.	Total pressure on the plate 	ii.	Position of centre of pressure.				  (10)

(OR)

17. 	a.	Derive an expression for the capillary rise.										    (5)

b.	A vertical cylinder of diameter 300 mm rotates concentrically inside another cylinder of diameter 310 mm. Both the cylinders are 250 mm high. The space between the cylinders is filled with a liquid whose viscosity is unknown. Determine the viscosity of the fluid if a torque of 15 Nm is required to rotate the inner cylinder at 120 rpm.					  (10)

18. 	a.	Derive the Bernoulli’s equation for a streamline flow from the Euler’s equation. 		    (8)

b.	In a two dimensional incompressible flow, the fluid velocity components are given by 		u=x – 4y and v= –y – 4x. Show that velocity potential exists and determine its form as well 	as stream function.															    (7)

(OR)

[P.T.O.]

19. 	a.	In a smooth inclined pipe of uniform diameter 250 mm, a pressure of 50 kN/m2 was 	observed at section 1 which was at elevation 10 m. At another section 2 at elevation 12 m, 	the pressure was 20 kN/m2 and the velocity was 1.25 m/s. Determine the direction of flow 	and the head loss between these two sections. The density of water at 20°C is 998 kg/m3.  								  												    (7)

b.	Derive the continuity equation for three dimensional flow.		   					    (8)

20. 	a.	A submarine fitted with a pitot tube moves horizontally in sea. Its axis is 12m below the 	surface of water. The pitot tube fixed in front of the sub-marine and along its axis is 	connected to the two limbs of a U-tube containing mercury, the reading of which is found 	to be 200 mm. Find the speed of the sub-marine. Take the specific gravity of sea water as 	1.025.						    	   											    (6)

b.	A horizontal venturimeter with inlet diameter 300mm and throat diameter 150 mm is employed to measure the flow of water. The pressure at inlet is 0.18 N/mm2 and the vacuum pressure at the throat is 300 mm mercury. Find the rate of flow. The value of coefficient of discharge may be taken as 0.98.				   					    (9)

(OR)

21. 	a.	A rectangular notch has a discharge of 0.48 m3/s, when head of water is 0.8 m. Find the 	length of the notch. Assume coefficient of discharge as 0.6.	   					    (5)

b.	Derive the Hagen Poiseullie equation for laminar flow through pipe. 				  (10)

22. 	a.	What are the types of the minor losses of energy?				 					    (6)

b.	Three pipes of diameters 300 mm, 200 mm and 400 mm and lengths 450 m, 255 m and

315 m respectively are connected in series. The difference in water surface levels in two tanks is 18m. Determine the rate of flow of water if coefficients of friction are 0.0075, 0.0078 and 0.0072 respectively considering: 		i.	Minor losses also 	ii.	neglecting minor losses.							  										    (9)

(OR)

23. 	a.	A pipe of diameter 300 mm and length 3500 m is used for the transmission of power by 	water. The total head at the inlet of the pipe is 500 m. Find the maximum power available 	at the outlet of the pipe, if the value of f = 0.006.	    								    (7)

b.	Three pipes of lengths 800 m, 500 m and 400 m and of diameters 500 mm, 400 mm and 	300 mm respectively are connected in series. These pipes are to be replaced by a single 	pipe of length 1700 m. Find the diameter of the single pipe.						    (8)

24. 	a.	What are the applications of Reynolds number and Froude number?				    (5)

b.	A ship 300 m long moves in sea water, whose density is 1030 kg/m3, A 1:100 model of this 	ship is to be tested in a wind tunnel. The velocity of air in the wind tunnel around the model is 30 m/s and the resistance of the model is 60 N. Determine the velocity of ship in sea water and also the resistance of the ship in sea water. The density of air is given as 1.24 kg/m3. Take the kinematic viscosity of sea water and air as 0.012 stokes and 0.018 stokes respectively																	  (10)

(OR)

25. 	Derive on the basis of dimensional analysis suitable parameters to present the thrust developed by a propeller. Assume that the thrust P depends upon the angular velocity ω, speed of advance V, diameter D, dynamic viscosity µ, mass density ρ, elasticity of the fluid medium which can be denoted by the speed of sound in the medium C.