Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	MECHANICS OF DEFORMABLE BODIES II			      Time : 3 hours

Subject Code:	09CE206										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the conditions for fixed support.

2.	When Newmarks’ method will be preferred?

3.	Differentiate long and short column.

4.	Write secant formula.

5.	Define thick cylinder.

6.	What is shrink fit?

7.	When shear centre coincides with centroidal axis?

8.	What is a curved beam?

9.	What is a straight beam?

10.	What is the Principal Moment Inertia for a rectangular section b x d.

PART – B  (5 x 3 = 15 MARKS)

11.	State Moment Area Theorem I.

12.	Write down the Effective Length for different end conditions.

13.	State the maximum Principal Strain Theory.

14.	Define Shear Centre.

15.	State the conditions for unsymmetrical bending.

PART – C  (5 x 15 = 75 MARKS)

16.	A beam AB of length L simply supported at the ends, carries a point load W at the distance ‘a’ from the left end. Find the deflection under the load and also the maximum deflection.

(OR)

17.	A cantilever of  length 6m carries a udl of 10 kN/m over the whole length. If E=200x106 kN/m2 and I= 30x10-5m4 using Conjugate Beam method, find the slope and deflection at the free end.

18.	From the following data, determine thickness of cast iron column:

Length of column=6m, External diameter=200mm, Load=500kN and FOS=6

Assume fixed ends and Ultimate Compressive stress and constant for hinged ends as 570MN/m2 and 1/1000 respectively.

(OR)

19.	a.	For long column with one end fixed and the other end free, find the Euler’s load.		(7)

b.	A solid round bar 16mm in diameter and 2.5m long is used as a strut. One end of the strut is fixed, while its other end is hinged. Find the safe compressive load for the strut, using Euler’s formula.

Assume E=200GN/m2 and FOS=3												(8)

[P.T.O.]

20.	A thick walled close end cylinder is made of an Aluminum Alloy (E= 72 GPa, 1/m=0.33), has inside diameter of 200mm and outside diameter of 800mm. The cylinder is subjected to internal pressure of 150 MPa. Determine the principle stress and maximum shear stress at a point on the inside surface of the cylinder. Also determine the increase in inside diameter due to fluid pressure.

(OR)

21.	A cylindrical shell made of mild steel plate and 1.2m in diameter is to be subjected to an internal pressure of 1.5MN/m2. If the material yields at 200MN/m2, calculate the thickness of the plate on the basis of the following 3 theories, assuming a FOS=3 in each case.

a.	Maximum Principal Stress Theory.

b.	Maximum Shear Stress Theory.

c.	Maximum Shear Strain Energy Theory.

22.	Determine the shear center of thin walled mono symmetrical I section having mean depth of 120mm; flange width on one side is equal to 60mm, the flange width on the other side being 30mm. The thickness of the section is uniform throughout and is equal to 3mm.

(OR)

23.	The curved member shown in the figure has a solid circular cross section 0.1m in diameter. If the maximum tensile and compressive stress in the member is not to exceed 150 MPa and 200MPa respectively, determine the value of P that can be safely carried by the member.

24.	A cantilever of I section is used to support the loads as shown in the figure. Determine the stress at corner A and position of neutral axis.

(OR)

25. 	An 80 mm x 80 mm x 10 mm angle section shown in the figure is used as simply supported beam over a span of 2.4 m. It carries a load of 400 N along the line YG, where G is the centroid of the section. Calculate:

a.	Stresses at the points A, B and C of the mid-section of the beam.

b.	Deflection of the beam at the mid-section and its direction with the load line.

c.	Position of the neutral axis.