Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	SURVEYING											      Time : 3 hours

Subject Code:	09CE207										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define surveying

2.	What is meant by Bench Mark?

3.	Name any two methods for measuring horizontal angle.

4.	What is meant by transiting of theodolite?

5.	State the advantages of anallatic lens.

6.	Define tacheometry.

7.	Define sight distance.

8.	Classify vertical curve.

9.	State the use of tower.

10.	Define signal.

PART – B  (5 x 3 = 15 MARKS)

11.	State the principles of surveying.

12.	What are various the temporary adjustments to be done for a theodolite?

13.	Compare fixed and movable hair method.

14.	State the requirements of transition curve.

15.	What is meant by reduction to centre.

PART – C  (5 x 15 = 75 MARKS)

16. 	The following readings were taken successively with a level .The instrument being shifted after third, sixth and eighth readings. The readings are 2.228, 1.606, 0.988, 2.090, 2.864, 1.262, 0.692, 1.982, 1.044, 2.684m. Enter the above readings in a level book and calculate the reduced level at all points if R.L. of first point is 432.384m.  Apply usual checks

(OR)

17.	State the various characteristics and uses of contour.

18.	a.	Explain any one method for distributing closing error in a traverse.      				    (7)

b.	Explain how you will measure vertical angle.                                       				    (8)

(OR)

19.	The following readings are obtained in a traverse survey, where the length and bearing of the last line were not recorded.

| Line | Length (m) | Bearing |
| --- | --- | --- |
| AB | 75.5 | 30O24I |
| BC | 180.5 | 110O36I |
| CD | 60.25 | 210O30I |
| DA | --- | --- |

Compute the length and bearing of line DA.

[P.T.O.]

20. 	The following observations were made using tacheometer with a multiplying constant 100.

| Instrument Station | Staff Station | Vertical angle | Stadia hair readings |
| --- | --- | --- | --- |
| P | BM | - 6O12I | 0.963,1.515,2.067 |
| P | Q | 7O5I | 0.819,1.341,2.067 |
| Q | R | 12O27I | 1.860,2.445,3.030 |

Calculate the reduced level of Q. Assume R.L. of B.M. as 460.50 m.

(OR)

21.	a.	Explain how subtense bar is used with a theodolite to determine the horizontal distance 			between two points.															    (7)

b. 	Two observations were taken with a transit theodolite, the reduced level of trunnion axis being 160.95m. In first case the angle of elevation was 4O36I and the staff reading 0.75m. In second case the angle of elevation was 5O48I and the staff reading 3.45. Calculate the horizontal distance between the instrument station and staff station      and also the R.L. of staff station																	    (8)

22. 	Explain the various surveys to conduct route surveys for railways.

(OR)

23. 	Name the various methods for setting out simple curve. Explain any two with neat diagram.

24. 	The following reciprocal observations were made from two points P and Q.  Horizontal distance PQ = 33128 m

Angle of depression of Q at P = 6’ 20”

Angle of depression of P at Q = 8’ 10”

Height of signal at P = 4.87m

Height of signal at Q = 4.07m

Height of instrument at P = 1.27m

Height of instrument at Q = 1.34m

Calculate the level difference between P and Q.

R Sin 1” = 30.88m	Assume suitable data

(OR)

25. 	a.	The altitude of two proposed stations A and B 100 km apart are respectively 420 m  and 	700 m. The intervening obstruction situated at C 70 km from A has an elevation of 478 m. 	Ascertain if A and B are intervisible and if necessary find by how much B should be raised 	so that line of sight must be no where less than 3m above the surface of the ground.     (8)

b.	What the various points to be kept in mind before selecting triangulation station.		    (7)