Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	STRUCTURAL ANALYSIS -I							      Time : 3 hours

Subject Code:	09CE208										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Determine the degree of static indeterminacy of the structure shown in the figure.

2.	State Betti Maxwell’s theorem.

3.	What do you mean by absolute maximum bending moment in a beam?

4.	What is the use of an influence line diagram?

5.	What do you mean by a linear arch?

6.	What is the degree of static indeterminacy of a three hinged parabolic arch?

7.	Define the term ‘tension coefficient’ used in the analysis of a space truss.

8.	Differentiate a plane truss and a space truss.

9.  	What is the use of three moment theorem?

10.	What do you mean by lack of fit in trusses?

PART – B  (5 x 3 = 15 MARKS)

11.	State and explain Castigliano’s Second theorem.

12.	What is the difference between a curve of S.F. or B.M. and an influence line of S.F. or B.M.?

13.	A parabolic arch hinged at the springings and crown has a span 20 m. The central rise of the arch is 4 m. It is loaded with a uniformly distributed load of intensity 2 kN/m on the left 8 m length. Calculate the magnitude of reactions a hinges.

14.	The horizontal span of a parabolic suspension cable is 100 m and the central dip is 8 m. Find the length of the cable.

15.	Write and explain the basic equation for Clapeyron’s theorem of three moments.

PART – C  (5 x 15 = 75 MARKS)

16.	Find the vertical and horizontal deflections of the joint C of the pin jointed truss as shown below. The area of the horizontal member is 150 mm2 and the areas of the members AC and BC are 200 mm2. Take E = 200 kN/ mm2.

[P.T.O.]

(OR)

17.	Calculate the slope at the supports and deflection at the mid span of a simply supported beam carrying a uniformly distributed load ‘w’ per metre length over the entire span.

18.	A UDL of 60 kN/m and length 5 m rolls over a girder of span 16 m. Find the maximum positive and negative shear force, maximum BM at a section 6 m from the left end.

(OR)

19.	Four point loads of 100 kN, 150 kN, 120 kN and 110 kN, spaced 2m between consecutive loads move on a girder of 20 m span, from left to right with the 100 kN load leading. Calculate the maximum bending moment at a point 10 m from left support. Also calculate the position and value of the absolute maximum bending moment.

20.	A three hinged parabolic arch of span 20 m and rise 4 m carries a UDL of 20 kN/m on the left half of the span. Find the maximum BM for the arch.

(OR)

21.	A two hinged parabolic arch of span 40 m and rise 8 m carries a point load of 80 kN at a distance of 10 m from the left support. Find the horizontal thrust at each support and the maximum BM.

22.	Determine the forces in the members of the truss shown in the figure.  AE is constant for all the members.

(OR)

23.	A suspension cable of span 150 m and dip 15 m carries a uniformly distributed load of 6 kN/m of horizontal span over the full span. Find the vertical and horizontal forces transmitted to the supporting pier if the cable is passed over smooth rollers on the top of the pier. The anchor cable makes 30° to the horizontal.

24.	Analyze the continuous beam ABC, shown in the figure and draw the BM diagram.

(OR)

25.	Find the force in all members of the truss shown in the figure if the member BC is short in length by 15 mm and is forced into position. Assume E = 2.1 x 105 N/mm2. All members have same area of cross section of 150 mm2.