Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title:	REINFORCED CONCRETE STRUCTURES – I			       Time: 3 hours

Subject Code:	09CE210	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What grade is used for nominal mix concrete?

2.	What is factored load?

3.	Define effective depth.

4.	What is doubly reinforced beam?

5.	Differentiate long and short column.

6.	Name the different types of column based on the types of loading.

7.	How will you calculate the nominal shear stress in beams?

8.	Write the formula to find the width of flange of T beam.

9.	What is two-way slab?

10.	What is the minimum eccentricity for the axially loaded column?

PART – B (5 x 3 = 15 MARKS)

11.	Find the moment of resistance of a singly reinforced beam of 175 x 350mm. Grade of concrete is M15.

12.	Show that the limiting depth of neutral axis for a rectangular cross section reinforced with Fe415 grade steel is 0.48d.

13.	A 4m high column is effectively held in position at both ends and restrained against the rotation at one end. Its diameter is restricted to 40cm, Find the slenderness ratio and minimum eccentricity.

14.	With neat sketches, write the various modes of shear failure in R.C Structures.

15.	A short column 300mm x300mm in section is reinforced with 8 bars of 22mm diameter. Find the safe load on the column by simple elastic theory. Take m=13.33.Use M20 mix and Fe 250 steel.

PART – C (5 x 15 = 75 MARKS)

16.	Determine the depth of neutral axis for the section shown below if

fck =15N/mm2, fy=250N/mm2

fck =20N/mm2,fy =415N/mm2

200mm

450mm

3-20dia

(OR)

[P.T.O]

17.	Design a rectangular beam for 4m effective span, which is subjected to dead load of 15KN/m and a live load of 12KN/m. Use M15 mix and Fe250 grade steel.

18.	Design a two-way slab for a room 5.5m x 4m clear in size, if the super imposed load is 5KN/m2.Use M15 mix and Fe250 grade steel. The edges are simply supported and the corners not held down.

(OR)

19.	Determine the shear stress in 25cm x 40cm effective rectangular section, if the shear force is 10KN and torsional moment is   2KNm at factored loads. Assume M15 mix and 0.25% tension steel at the given section. State whether torsional reinforcement is required.

20.	Design a circular column to carry an axial load of 1500KN using lateral ties. Use M15 mix and Fe250 grade steel.

(OR)

21.	Design a reinforced concrete combined rectangular footings for 2 columns located 3.6m apart. The overall sizes of the columns are 400mm x400mm and 600mm x 600mm and the loads on them are 1000KN and 1500KN respectively. The space available for the width of the footing is restricted to 1.8m.The safe bearing capacity of the soil is 280KNper sqm. Use M15 mix and grade 1MSbars.

22.	An RC Tbeam with flange width of 1020mm has a tension steel area of 1884mm2. Taking the permissible stresses in concrete and steel as 7N/mm2 and 236N/mm2.Determine the moment of resistance of the beam. The thickness of the flange is 130mm and the effective depth of the beam is 510mm.The breadth of the rib is 275mm.

(OR)

23.	A beam 250mm wide and 500mm effective depth and 6m span supports a total load of 19,500N including its own weight. Find the maximum shear stress and determine the spacing of 10mm stirrups. Allowable shear stress is 0.5N/mm2

24.	A slab over room is 5m x5m.The edges of the slab are simply supported and the corners are not held down. The live load on the slab is 2500N/m2.The slab has a bearing of 150mm on the supporting walls. Design the slab as per Rankine-Grashoff Theory. Use M15 mix and Fe250 grade steel.

(OR)

25.	Design a short reinforced concrete column of circular section to carry an axial load of 2000KN. The column should have a lateral reinforcement in the form of a spiral. Use M15 mix and Fe250 grade steel.