Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	THEORIES AND PRACTICES OF WATER AND WASTEWATER ENGINEERING										     Time : 3 hours

Subject Code:	09CE211										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Maximum daily demand is  ___________times average daily demand.

2.	What is meant by coincident draft?

3.	What is meant by MPN?

4.	Define sewage.

5.	Mention the uses on nomograms.

6.	Mention the common types of pumps used in water supply system.

7.	The available pressures at the fire hydrant is ___________ when direct flow is applied.

8.	What is meant by gravitational system?

9.	Mention the uses of egg shaped sewer.

10.	What is meant by lamp hole?

PART – B  (5 x 3 = 15 MARKS)

11.	What is meant by design period?

12.	Distinguish between BOD and COD.

13.	Mention the different materials used in pipes.

14.	Draw the grid iron system.

15.	What is meant by Siphon?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the different methods of forecasting future population of a city for which a water supply scheme is to be planned.

(OR)

17.	What is meant by water quality standards? Mention the important water quality parameters with its permissible limits.

18.	Explain the factors on which the sanitary sewage flow depends on.

(OR)

19.	Explain the empirical methods used for estimating storm water flow.

20.	Estimate the hydraulic gradient in a 2 m dia. Smooth concrete pipe carrying a discharge of 3 cumecs at 1000 C temperature by using:		a.	Darcys formula 		b.	Mannings formula c.	Hazen – Williams formula. Assume suitable data.

(OR)

21.	Write a detail note on pipe appurtenances.

22.	Illustrate with sketches the different types of layouts of pipe systems in distributing water and compare the merits and demerits.

(OR)

23.	With a neat sketch explain the layout of house connection system.

[P.T.O]

24.	Determine the size of a circular sewer for a discharge of 650 l/s running half full. Assume n=0.015 and S=0.0001.

(OR)

25.	Explain one pipe system and two pipe system with neat sketches,