Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	DESIGN OF MASONRY AND TIMBER STRUCTURES	     Time : 3 hours

Subject Code:	09CE213										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Stability.

2.	What do you understand by the term Buoyancy?

3.	What is meant by non-load bearing wall?

4.	What is the standard size of a brick?

5.	Under what conditions stability of dam is tested?

6.	Define Bearing Capacity.

7.	Define Shear Wall.

8.	What is meant by a lintel band?

9.	Define the term Flitched Beam.

10.	What is Diagonal bracing?

PART – B  (5 x 3 = 15 MARKS)

11.	How structures are classified?

12.	Classify the various types of Masonry Walls.

13.	How stability of retaining wall is checked?

14.	What the factors influencing the behaviour of building during Earthquake?

15.	Explain the Construction of Wood Wall.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail about the Different Structural Systems.

(OR)

17.	Discuss in detail about any two structural materials. Also compare the choices between different materials.

18.	Determine the allowable axial load per m length of a (a) 200 mm solid wall (b) 240 mm cavity wall with 40 mm cavity. The height of the wall between thicker plinth wall and the slab is 3.5 m the wall is continuous and its length between cross wall is 5m. Take Fe =0.55 MPa. Effective height of wall = 0.85 X3.5 = 2.97m and effective length of the wall = 0.85X5.0 = 4.00 m

(OR)

19.	Design a brick column 3m high to carry an axial load of 110 kN. The width of the pier is limited to 11/2 normal brick size for architectural reasons. Assume cement-lime mortar 1:1:6 and first class brick with 10 MPa Strength. The column may be taken as fixed restrained.

20.	A retaining wall, vertical, at the earth face, is 6m tall, 2.1m wide at the base and 0.45 m wide at the top.  It retains an earth fill which is level with the top of the wall. If the sine of the angle of internal friction is 0.95, density of soil 1760 kg/m3 and weight of wall 23.5 kN/m3, find the critical stresses at base.

(OR)

[P.T.O.]

21.	The bottom width of a retaining wall is 3m.The resultant thrust has the normal component of 150 kN/m length of wall and crosses the bottom at a distance of (a) 400 mm (b) 600 mm from the center. Determine the intensity of maximum and minimum pressure on foundation.

22.	Describe the behaviour of under reinforced masonry and reinforced masonry wall.

(OR)

23.	Design a shear wall of length 3.0 and thickness 230 mm subjected to following forces. Assume Fck=25 N/mm2 and fy=415N/mm2

| Loading | Axial force in kN | Moment in kN/m | Shear in kN |
| --- | --- | --- | --- |
| DL+LL | 1950 | 600 | 20 |
| Seismic load | 250 | 4800 | 700 |

24.	Design timber beams and girders for the roof of a room having 6 m X 8 m clear dimensions. The timber is sal. The roof covering consists of timber planks with earth and brick tiles over them, so that the dead load of roofing is 2.5 kN/m2. The live load on the roof is 1.47kN/m2

(OR)

25.	A hall is 6 m wide and contains flitched timber beams spaced 3m apart. The dead and the effective live loads are 4.0 kN/m2 each. The section consists of two pieces of wood                12.5 cm x 30 cm actual size and a steel plate of 12mm X 20cm sandwitched between them. Find the maximum stress in timber and steel and also the maximum deflection at the center. E for steel 2.0 X105, E for wood 10000 N/mm2