Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	APPLIED HYDRAULICS AND FLUID MACHINES  		      Time : 3 hours

Subject Code:	09CE216										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Distinguish between Prismatic Channel and Non Prismatic Channel.

2. 	Define hydraulic jump.

3. 	Force exerted by fluid at right angles to the direction of flow is called as _______.

4. 	Write the formula for displacement thickness.

5. 	Define Impulse Momentum Principle.

6. 	The force exerted by a jet of water on a moving vertical plate, in the direction of motion of plate is given by _________.

7. 	Define specific speed of turbine.

8. 	What is draft tube?

9. 	Define slip of reciprocating pump.

10. 	What is a multi pump?

PART – B  (5 x 3 = 15 MARKS)

11. 	A 8m wide channel conveys 15m3/s of water at a depth of 1.2m. Calculate specific energy and

critical depth.

12. 	Explain briefly the terms boundary layer with at least two examples

13. 	A jet of water, 75 mm in diameter, issues with a velocity of 30 m/s and impinges on stationary

flat plate which destroys its forward motion. Find the force exerted by the jet on plate and

its work done.

14. 	Distinguish between Reaction turbine and Impulse turbine.

15. 	What is the need of air vessel in a reciprocating pump?

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Define the most economical section for an open channel and derive an expression for most 	economical rectangular channel.												(8)

b.	A trapezoidal channel having the side slope equal to 60o with the horizontal and laid on a

slope of 1 in750, carries a discharge of 10 m3/s. Find dimensions of most economical 	channel section. Take c = 66. 													(7)

(OR)

17. 	A 3.6 m wide rectangular channel conveys 90m3/s of water with a velocity of 6m/s. Is there a condition for hydraulic jump to occur? If so, calculate the height, length and strength of jump. What is loss of energy/kg of water?

18. 	a.	Explain Boundary layer separator and its control measures.						(5)

b.	The velocity distribution in the boundary layer is given byuU=yδ , where u=velocity at a

distance of y from the plate and u=U at y=δ,  δ being boundary layer thickness . Find

a.	Displacement thickness  	b.	Momentum thickness 		c.	Energy thickness

d.	Value of  δθ.															(10)

[P.T.O.]

(OR)

19.	a.	Derive the expression for drag and lift.											(7)

b.	A truck having a projected area of 6.5 m2 travelling at 70 km/hr has a total resistance of 	2000 N. Of this 20% is due to rolling friction and 10% is due to surface friction. The rest is

due to form drag. Make calculation for form drag. e=1.22 kg/m3 for air.				(8)

20. 	A 75 mm diameter jet having a velocity of 30 m/s strikes a flat plate, the normal of which is inclined at 45o two the axis of the jet. Find the normal pressure on the plate:

a.	when plate is stationary 	b. 	When plate is moving with a velocity of 15m/s, moving away from the jet, the normal force on the plate		c. 	W.d/sec  	d.	Power of jet.

(OR)

21. 	A jet of water of 50 mm diameter impinges on a curved vane and is deflected through an angle of 175o. The vane moves in the same direction of the jet with a velocity of 35 m/s. If the rate of flow is 170 lbs/sec. Determine the component of force on the vane in the direction of motion. How much would be the power developed by the vane and what would be the water efficiency. Neglecting friction.

22. 	The following data relates to a pelton wheel, Head = 72 m, speed of wheel = 340 R.P.M, shaft power of wheel 115 KW, speed ratio = 0.45, co efficient of velocity = 0.98, overall efficiency 85%. Design the pelton wheel.

(OR)

23.	a.	What are the points to be considered while selecting a hydraulic turbine for hydro electric 	power plant?																	(3)

b.	A reaction turbine works at 450 RPM under a head of 120 m. Its diameter at inlet is 1.2 m

and the flow area is 0.4 m2. The angles made by absolute and relative velocities at inlet are

20o and 60o respectively with tangential velocity.  Determine:

i.	volume flow rate		ii.	power developed 		iii.	Hydraulic efficiency		(12)

24. 	a.	With a neat sketch explain the working of an air vessel.							(6)

b.	A double acting reciprocating pump running at 40 rpm is discharging 1m3 of water/minute.

The pump has a stoke length of 400 mm, the diameter of the piston is 200 mm. The delivery and suction head are 20 m and 5 m respectively. Find the slip of pump and power

required to drive the pump. 													(9)

(OR)

25.	 A centrifugal pump impellor having external and internal diameter 480 mm and 240 mm respectively is running at 1000 R.P.M. the rate of flow through the pump is 0.0576 m3/s and velocity of flow is constant and is equal to 2.4 m/s. the diameter of suction and delivery pipes are 180 mm and 120 mm respectively and suction and delivery heads are 6.2 m (abs) and 30.2 m of water respectively. If the power required to the pump is 23.3 kw and the outlet vane angle is 45o. Determine:

a. 	Inlet vane angle 		b. 	Overall efficiency of pump 	c. 	Manometric efficiency of pump