Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	REINFORCED CONCRETE STRUCTURES - II			      Time : 3 hours

Subject Code:	09CE217										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the minimum live load to be taken on stairs liable to overcrowding?

2.	What is the purpose of providing shear key in a retaining wall?

3.	What is the design behaviour of a rectangular water tank with length to breadth ratio more than 2?

4.	What is staging in a water tank?

5.	For Class A and Class B loading, the impact factor for Reinforced Concrete Bridge is ____________.

6.	For solid slabs spanning in one direction, what is the moment to be considered for the distribution reinforcement?

7.   	For the wind load analysis of tall building, ___________ method is used.

8.	Define ductility of a structural member.

9.   	The ____________ propagate with the increase in deflection till the slab reaches failure.

10.  Draw the yield line pattern for a simply supported rectangular slab.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the distribution of loading on stairs.

12.  List out the load acting on an underground water tank stating its critical cases.

13.	State the conditions under which Courbon method can be adopted while designing longitudinal girders of a bridge.

14.	Write down the basic assumptions in the earthquake resistant design.

15.	Write down the characteristic features of yield lines.

PART – C  (5 x 15 = 75 MARKS)

16. 	Design the Open well staircase for a Public building of 3.75m height which is likely to be overcrowded. The rise and tread to be provided are 150mm and 250mm respectively. The dead weight of finishes may be taken as 0.1875 kN/m2 of the finished surface of the step.  The width of stair is 1.25 m and it is limited to 9 steps in a flight. The materials used are M20 grade concrete and HYSD steel of grade Fe415.

(OR)

17. 	Design a cantilever retaining wall stem to retain the earth embankment of 4m high above ground level.  The density of earth is 24 kN/m3 and its angle of repose is 28o.  The embankment is horizontal at its top.  The safe bearing capacity of soil is 220 kN/m2 and the coefficient of friction between soil and concrete is 0.8.  Adopt M20 concrete and Fe 415 steel.

18. 	Design a circular water tank with fixed base for capacity of 400000 liters. The depth of water is to be 4 m.  Free board is 200 mm. Use M20 grade concrete and Grade I mild steel.

(OR)

[P.T.O.]

19.  	Design the side walls of a square RCC tank of capacity 70000 liters.  Depth of water in the tank is 2.8m and the free board is 0.2 m.  Adopt M20 concrete and Grade I Steel.  Tensile stresses in steel are limited to 100 N/mm2 at water face and 125 N/mm2 away from face. Sketch the details of reinforcement in the walls of the tank.

20. 	Design a reinforced concrete slab bridge for a state highway to suit the following data;

Clear span – 3.5m; loading – IRC Class A; Number of lane – single; Road width – 3.8 m; Safety kerb width – 0.6 m; wearing coat – 80 mm thick.  Use M20 concrete and Grade I steel

(OR)

21. 	Using Courbon’s method, design the deck slab of RC T- beam girder bridge for a National Highway of clear width 7.5 m with c/c of bearing span as 16 m.  Use IRC Class AA tracked vehicle.  Assume 80 mm thick wearing coat.  Adopt M25 concrete and HYSD Fe415 steel.

22. 	Using Portal method, determine the moments in columns and beams.  Assume all columns are of equal area of cross section.

H

G

A

B

C

F

E

D

I

3m

4m

2m

4m

20kN

15kN

(OR)

23. a.	Explain in brief about the construction practices of reinforced concrete building and the 	lessons learnt from damages of RC buildings.									  (10)

b.	Give the steps involved to determine the equivalent seismic force on the building at each 	floor.																		    (5)

24. 	An Isotropically reinforced simply supported square slab subjected to a UDL wu/m2.  Find the ultimate value of wu for Mx = My = m

(OR)

25. 	Design the equilateral triangular slab of 3.5 m sides for the live load of 3 kN/m2 isotropically reinforced.  Use M15 concrete and Fe415 steel.  Assume the slab thickness as 100 mm.