Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	STRUCTURAL ANALYSIS-II   						      	     Time : 3 hours

Subject Code:	09CE218										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In a fixed-fixed beam if one support sinks by unit distance, what will be the moment induced at the supports?

2.	In a propped cantilever beam of single span, if the propped support rotates by unity, what will be the moment induced at fixed support?

3.	What is meant by Carry-over factor?

4.	What is meant by Distribution factor?

5.	Principle of superposition can be applied to non-linear elastic material. True or false?

6.	What is meant by a transformation matrix?

7.	Write any two properties of a stiffness matrix.

8.	What is a stiffness coefficient?

9.	Write the relation between time period and natural frequency.

10.	Write the equation of motion for a damped free vibration.

PART – B  (5 x 3 = 15 MARKS)

11.	List the terms in which the slope deflection equation expresses the final end moments of each member in a frame.

12.	Define absolute stiffness of a member.

13.	Write the force-displacement transformation matrix for a simply supported beam element.

14.	Eliminate u3 in the following equation by Static condensation technique.

398−198−52−198398−52−52−525u1u2u3=364

15.	What is meant by critical damping?

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the end moments of the members of the frame shown in figure. The relative EI values for each member are indicated along the members. Use Slope-Deflection method.

(OR)

[P.T.O.]

17.	A continuous beam is shown in figure. During loading support B sinks by 10 mm, determine the support moments. E = 200 x 106 kN/m2 and I = 80 x 10-6 m4. Relative I value for each span is indicated. Use slope deflection method.

18.	Determine the end moments of the members of the frame shown in figure. The relative EI values for each member are indicated along the members. Use Moment-Distribution method.

(OR)

19.	A continuous beam is shown in figure during loading support B sinks by 10 mm, determine the support moments. E = 200 x 106 kN/m2 and I = 80 x 10-6 m4. Relative I value for each span is indicated. Use Moment-Distribution method.

20.	Analyze the continuous beam shown in figure by flexibility method.

(OR)

21.	Develop the flexibility matrix for the structure shown in figure. EI is constant.

22.	Analyze the continuous beam shown in the figure by stiffness method.

(OR)

23.	Generate the stiffness matrix for the system shown in the figure.

24.	Consider the frame shown in the figure. This is a rigid steel frame to which a horizontal dynamic force is applied at the upper level. As part of the overall structural design it is required to determine the natural frequency of the frame. Assume: the masses of the walls and columns are negligible and the girder is sufficiently rigid to prevent rotation at the top of the columns.

I = 82.5 in4.

(OR)

25.	Determine the natural frequency of the system shown in figure. Consisting of a weight of

50.7 lb attached to a horizontal cantilever beam through the coil spring k2. The cantilever beam has a thickness of ¼ in, width 1 in, E = 30 x 104 psi, L = 12.5 in and k2 = 10.69 lb/in.