Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	DESIGN OF STEEL STRUCTURES  (LIMIT STATE METHOD)   														      						Time : 3 hours

Subject Code:	09CE219							      		       	Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the types of welds?

2.	What are the uses of bolted connection?

3.	Draw the various forms of tension members.

4.	What is a strut?

5.	What is local buckling?

6.	Classify the welded beam connections.

7.	___________ is economical for large span and heavy gravity loads.

8.	What is the purpose of providing intermediate vertical stiffeners in plate girder?

9.	Slope of a truss is equal to its ___________.

10.	How the self -weight of a root truss (N/m2) can be obtained?

PART – B  (5 x 3 = 15 MARKS)

11.	Design a single bolted double cover butt joint to boiler plates of thickness 12mm for maximum efficiency. Use M16 bolt of grade 4.6. Boiler plates are of Fe 410 grade. Find the efficiency of the joint.

12.	Determine the design tensile strength of the plate 200mm X 12mm with the holes for 16mm diameter bolt as shown in figure

13.	Write the steps involved in the design on laterally supported beam.

14.	Write the steps involved in the design of plate girder.

15.	What are the loads acting on root truss?

PART – C  (5 x 15 = 75 MARKS)

16.	Design a suitable bolted bracket connection of a ISHT-75 section attached to the flange of a ISHB 300 at 577N/m to carry a vertical factored load of 600kN at an eccentricity of 300 mm. Use M24 bolts of grade 4.6.

(OR)

17.	A tie member of a root truss consist of 2 ISA 100 x75.8mm. The angles are connected to either side of a 10mm gusset plates and member is subjected to a working pull of 300KN Design the welded connection. Assume connection are made in the workshop.

18.	Design a double angle tension member connected on each side of a 10mm thick gusset plate, to carry an axial load of 375 KN. Use 20mm black bolt. Assume shop connection.

(OR)

19.	A column 4 m long has to support a factored load of 6000KN. The column is effectively held at both ends and restrained in direction at one of the ends. Design the column using beam section and plates.

[P.T.O.]

20.	An ISMB 500 section is used as a beam over a span of 6 m , with simply supported ends. Determine the maximum factored uniformly distributed load that the beam can carry if the ends are restrained against torsion but compression flange is laterally unsupported.

(OR)

21.	Design a simply supported beam of effective span 1.5 m carrying a factored concentrated load of 360KNat mid span.

22.	Design a welded plate girder of span 30m to carry on carry on superimposed load of 35KN/m. Avoid use of bearing and intermediate stiffeners. Use Fe 415(E250) steel.

(OR)

23.	Design the plate girder of span 30m to carry on carry on superimposed load of 35KN/m using thin web and stiffener, but avoid intermediate stiffeners.

24.	Determine the design loads on the purlins of an Industrial building near Visakhapatnam. Given:Class of Building : General with life of 50 years, Terrain: Category 2,Maximum Dimension :40 m, Width of Building :15m,Height at eye level: 8m,Topography:θ less than 3o, Permeability : Medium, Span of truss :15m,Pitch:1/5,Sheeting:A.C.Sheets,Spacing of purlins:1.35m,Spacing of trusses:4m.

(OR)

25.	Design a truss of span 15m, spacing 4 m to be built near Visakhapatnam, other details being same as given in Question No.24.