Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	FOUNDATION ENGINEERING   						      Time : 3 hours

Subject Code:	09CE221										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give any two assumptions in Boussinesq's theory of stress distribution.

2.	Name a field test to determine bearing capacity of soil.

3.	Define Area ratio.

4.	Name two samplers used to collect undisturbed soil samples.

5.	What is the difference between a shallow and deep foundation?

6.	How are caissons classified based on construction?

7.	Define active earth pressure.

8.	Give an example of earth pressure at rest condition.

9.	Define optimum moisture content.

10.	What are the various methods of field compaction?

PART – B  (5 x 3 = 15 MARKS)

11.	What is a pressure bulb?

12.	When do you apply corrections to standard penetration number? Elaborate.

13.	What is a compensated raft foundation?

14.	When do you suggest a counterfort retaining wall?

15.	What is the difference between light compaction test and heavy compaction test?

PART – C  (5 x 15 = 75 MARKS)

16.	What is the significance of Influence charts? Explain the construction of Newmarks influence chart.

(OR)

17.	What are the assumptions in Terzaghis bearing capacity analysis? A square footing located at a depth of 1.5m from the ground surface carries a load of 150kN. The soil is submerged having an effective unit weight of 11kN/m3 and angle of shearing resistance 300. Find the size of the footing assuming factor of safety =3 Nq=10 Nr =6.0.

18.	Explain any one method of geophysical investigation along with the advantages and disadvantages

(OR)

19.	Explain standard penetration test along with the interpretation of results.

20.	What is group efficiency of piles? A group of 4 x 4 piles is driven in dense sand layer 2.5m thick. Pile spacing is 1.0 m c/c and diameter of piles is 0.4m. Total load on top is 3000 kN.           A compressible layer of clay 2m thick exists below dense sand. Clay is underlain by rock. Find the settlement of pile group if coefficient of volume compressibility of clay layer is                1.5x 10-4m2/kN.

(OR)

[P.T.O.]

21.	What are caissons? Explain the construction of pneumatic caisson.

22.	Explain any one graphical method of determination of earth pressure

(OR)

23.	A 4.5 m high retaining wall is restrained from yielding retains sand of angle of internal friction 300. The water table is at a depth of 3m from the top of the backfill. The unit weight of sand above and below the water table are 16kN/m3 and 19.81 kN/m3 respectively. Find the force on the wall per unit length. Also determine the force when wall moves away from the fill.

24.	What are the various ground improvement techniques? Explain any one method of stabilization of soil.

(OR)

25.	Explain sand replacement method along with its significance.