Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	FLUID MECHANICS AND MACHINERY				      Time : 3 hours

Subject Code:	09CE242										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define viscocity.

2.	State Pascal’s law

3.	What is meant stream line?

4.	Write the continuity equation.

5.	Mention the uses of flow meters.

6.	Define water hammer.

7.	Give an example for roto dynamic pump.

8.	What is meant by negative slip?

9.	Define specific speed.

10.	What is meant by velocity triangle?

PART – B  (5 x 3 = 15 MARKS)

11.	State Newton’s law of viscosity.

12.	Distinguish between rotational and irrotational flow.

13.	Write a note on minor losses through pipes.

14.	What is meant by indicator diagram?

15.	Classify turbines based on specific speed and type of energy.

PART – C  (5 x 15 = 75 MARKS)

16.	What is a manometer? Explain the difference between U tube manometer and inverted U tube manometer with its applications.

(OR)

17.	A U tube differential manometer connects two pressure pipes A and B. Pipe A contains carbon tetra chloride having a specific gravity 1.594 under a pressure of 11.772 N / cm2 and pipe B contains oil of specific gravity 0.8 under pressure of 11.772 N / cm2. The pipe A lies 2.25 m above pipe B. Find the difference of pressure measured by mercury as fluid filling U tube.

18.	Write Euler’s equation of motion along a stream line and integrate it to obtain Bernoulli’s equation. State all assumptions made.

(OR)

19.	Determine the velocity of flow at radii of 0.2m, 0.4m and 0.8m, when the water is flowing radially outward in a horizontal plane from a source at a strength of 12 m2/sec.

20.	Find the discharge over a rectangular weir of length 100m. The head of water over the weir is 1.5 m. The velocity of approach is given as 0.5 m/s. Take Cd = 0.6.

(OR)

21.	Derive an expression for the loss of head due to: 		a.	Sudden enlargement

b.	Sudden contraction of a pipe.

[P.T.O.]

22.	A jet of water of 2.5 cm diameter, moving with a velocity of 10 m/sec, strikes the hinged square plate of weight 98.1 N at the center of the plate. The plate is of uniform thickness. Find the angle through which the plate will swing.

(OR)

23.	With a neat sketch explain components of reciprocating pump.

24.	Define the specific speed of a turbine. Derive an expression for the specific speed. What is the significance of the specific speed?

(OR)

25.	Write short notes on: 		a.	Surge Tank 		b.	Draft tube 	c.	Performance curves.