Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	MECHANICS OF SOLIDS								      Time : 3 hours

Subject Code:	09CE245										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define strain.

2.	Define factor of safety.

3.	Define bending moment.

4.	Define shear force.

5.	Write the relationship between bending stress and radius of curvature.

6.	Write simple bending equation.

7.	What will be the maximum shear stress by Mohr’s circle method?

8.	What is strain energy?

9.	Write the expression for deflection at free end of a cantilever beam with udl over whole span.

10.	What is meant by crippling load?

PART – B  (5 x 3 = 15 MARKS)

11.	State Hooks law.

12.	What will be the maximum bending moment for a simply supported beam of length 6m carrying an udl of 6 kN/m?

13.	State the assumptions in theory of simple bending.

14.	Define principal stress and principal planes.

15.	Write the expression for crippling load of a column:		a.	when one end is fixed and other end is free. 		b.	both ends are fixed.

PART – C  (5 x 15 = 75 MARKS)

16. 	A bar of 30 mm dia is subjected to a pull of 60 kN. The measured extension on a gauge length of 200 mm is 0.09mm and change in diameter is 0.004mm. Calculate the Poisson’s ratio, Young’s modulus and Bulk modulus.

(OR)

17.  	Derive an expression for hoop stress and longitudinal stress of a thin cylindrical shell subjected to an internal pressure.

18. 	A cantilever 1.5 m long is loaded with uniformly distributed load of 2 KN/m run over the length of 1.25 m from the free end. It also carries a point load of 3 KN at a distance of 0.25 m from the free end. Draw the shear force and bending moment diagram.

(OR)

19. 	Draw the shear force and bending moment diagram for a simply supported beam of length 9 m and carrying a uniformly distributed load of 10 KN/m for a distance of 6m from the left end. Also calculate the maximum bending moment on the section.

20. 	a.	Derive an expression for strain energy due to bending.       						    (8)

b.	A leaf spring is to be made of 7 steel plates 65mm wide and 6.3mm thick. Calculate length of spring so that it may carry a central load of 2750N, stress being limited to 160 N/mm2. 																				    (7)

[P.T.O.]

(OR)

21. 	a.	Write the assumptions in theory of pure torsion.									    (5)

b.	In a hollow circular shaft of outer and inner diameters of 20 cm and 10 cm respectively, the 	shear stress is not to exceed 40N/ mm2. Find the maximum torque the shaft can safely 	transmit.  																	  (10)

22. 	The principal tensile stresses at a point across 2 perpendicular planes are 80 N/mm2 and            40 N/mm2. Find normal and tangential stress and resultant stress and its obliquity on a plane at 20 degrees with major principal plane.

(OR)

23. 	Explain maximum principal stress theory and maximum strain energy theory.

24.	A beam of length l simply supported at ends carries a point load W at a distance a  from left end. Find the deflection under the load using Macaulay’s method.

(OR)

25.	Find Euler’s critical load for a hollow cylindrical cast iron column 200 mm external dia and      25 mm thick, if it is 6m long and hinged at both ends. Take E =  8 x 104 N/mm2