Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ENGINEERING MECHANICS							      Time : 3 hours

Subject Code:	09CE247										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.  	What is the dimension of power?

2.  	Define the term free body.

3.  	What is couple?

4.  	Give the ratio of tension in tight side to slack side.

5.  	Moment of inertia of a circular area about an axis perpendicular to the area is ___________.

6.  	What is radius of gyration?

7.  	Define kinematics.

8.  	A satellite orbits around the earth due to ___________.

9.  	If both mass, acceleration of the block are doubled, the force acting on the mass with respect to

the previous value increases ___________.

10. 	What is impulse force?

PART – B  (5 x 3 = 15 MARKS)

11. 	State polygon law of forces.

12. 	List the different types of loads.

13. 	Distinguish between centroid and centre of gravity.

14. 	What do you mean by projectile?

15. 	State the principle of work and energy.

PART – C  (5 x 15 = 75 MARKS)

16. 	AD is a string hung from a horizontal ceiling at A and D. A weight of 75 N is hung from point

B. Determine the magnitude of weight that should be hung from point C such that the portion

BC of the string is horizontal. Point B is 0.6 m from A and point C is 0.35 m from D. Also

Calculate the tension in various portions AB, BC and CD of the string.

(OR)

17. The x, y, z components of a force are 36 kN, –24 kN and 24 kN respectively. Find the 	component of this force along the line joining A (1, 2, –3) and B (–1, –2,2).

18. 	Find the support reactions of the simply supported beam of span 8 m. It carries a point load      	of 80 kN, 5m from left support and another point load of 60 kN, 1.5 m from right support.

(OR)

19. 	A block weighing 800 N lying on a horizontal floor is just dragged by a force P inclined at 350  	to the floor. Find the value of P and the inclination of P with horizontal, so that P is minimum 	and the value of Pmin. Assume co-efficient of friction = 0.25.

20. 	Explain the procedure of analysis of roof truss by method of joints with suitable example.

(OR)

21. 	State and prove parallel axis theorem and perpendicular axis theorem.

[P.T.O.]

22. 	A body moving with uniform acceleration is observed to travel 33 m in eighth second and       53 m in the thirteenth second of its travel. Calculate the velocity at start and uniform acceleration.

(OR)

23. 	A shot is fired with a bullet with an initial velocity of 40 m/s from a point 20 m in front of a

vertical wall 10 m high. Find the angle of projection with horizontal to enable the shot to just

clear the wall.

24. 	A wagon of mass 3000 kg is pushed along the rails with a constant force of 500 N.  If the resistance due to friction is 125 N, determine the time taken for the wagon to reach a speed of  6 m/s.

(OR)

25. 	A lift has an upward acceleration of 2.5 m/s2. What pressure will a man of weight 800 N exert on a floor of the lift? Determine the pressure he would exert if the lift has an acceleration of  2.5 m/s2 downwards. Also determine the upward acceleration to cause the weight to exert a

pressure of 1200 N on the floor. Assume g = 9.81 m/ s2.