Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	BASIC ELECTRONICS									      Time : 3 hours

Subject Code:	09EC218										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	At room temperature the current in an intrinsic semiconductor is due to

a.	holes		b.	electrons	c.	ions			d.	holes and electrons

2.	In an n channel JFET, the gate is

a.	n type		b.	p type		c.	either n or p	d.	partially n & partially p

3.	In the op-amp circuit of figure, V0 =

a.	Vi		b.	3 Vi		c.	5 Vi		d.	0.5 Vi

4.	If ground is applied to the (+) terminal of an inverting op-amp, the (–) terminal will:

a.	not need an input resistor 			b.	be virtual ground

c.	have high reverse current			d.	not invert the signal

5.	Binary number 101101 when converted to its 2’s complement will become :

a.	0101.01 							b.	0100.11

c.	0100.10 							d.	0101.10

6.	Combinational circuit has :

a.	memory 							b.	no memory

c.	flip-flops 						d.	counters

7.	A carrier is simultaneously modulated by two sine waves having modulation indices of 0.4 and 0.3. The total modulation index will be

a.	0.1								b.	0.7

c.	0.5								c.	0.35

8.	The c(t) and m(t) are used to generate an FM signal. If the peak frequency deviation of the generated FM signal is three times the transmission bandwidth of the AM signal, then the coefficient of the term cos [2π(1008 x 103t)] in the FM signal (in terms of the Bessel Coefficients) is

a.	5 J4(3)		b.	52J8(3)		c.	52 J8(4)		d.	5 J4(6)

9.	An FM transmitter has maximum frequency deviation of 75 kHz and reproduces audio signals up to 15 kHz. Minimum channel width required is __________ kHz.

a.	180			b.	120			c.	90			d.	60

10.	Rotation of a geosynchronous satellite means its

a.	drift from stationary position		b.	wobbling

c.	three-axis stabilization			d.	three-dimensional stabilization

[P.T.O.]

PART – B  (5 x 3 = 15 MARKS)

11.	Compare JFET with BJT.

12.	List out the ideal characteristics, and draw the equivalent diagram of an OP-AMP.

13.	Simplify the following expression using Boolean laws.

Y = (B + BC)(B + B′C)(B + D)

14.	State the role of modulation index in AM and FM systems.

15.	Define EIRP. Also mention its limitation.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Illustrate the working of PN junction diode under forward bias and reverse bias condition. 																					    (8)

b.	Describe the operation of UJT and its emitter characteristics.						    (7)

(OR)

17.	With neat diagram, illustrate the operation of a Depletion mode MOSFET and its comparison over enhancement MOSFET.

18.	a.	Derive the slew rate equation for an op-amp.										    (7)

b.	Suggest and brief the method of improving the CMRR using active load.			    (8)

(OR)

19.	Illustrates the steps involved in monolithic IC Fabrication with suitable pictorial representation.

20.	a.	List the salient features of the BCD, Excess-3 and Gray codes.						    (7)

b.	Minimize the following logic equation											    (8)

Y = A′ B′ C D + A′ B C D′ + A′ B′ C′ + A′ B′ D′ + A′ C′ + B′

(OR)

21.	a.	Realize the full subtractor circuit using NAND gates only.							    (7)

b.	Implement the following logic function using 4:1 multiplexer.						    (8)

F (A, B, C, D, E) = ∑ m (0, 1, 2, 3, 6, 8, 9, 10, 13, 15, 17, 20, 24)

22.	a.	Narrate an AM generation using balanced modulator.								  (10)

b.	A radio telephone transmitter using AM has unmodulated carrier output of 20 kW and can be modulated to a maximum depth of 80% but a sinusoidal modulating voltage without causing overloading. Find the value to which unmodulated carrier power may be increased without resulting in overloading if the maximum permitted modulation index is restricted to 60%.																		    (5)

(OR)

23.	a.	Brief the demodulation of  FM using Balanced Slope Detector.						  (10)

b.	When the modulating frequency in a FM system is 400 Hz, modulating voltage is    2.4 V and the modulation index is 60. Calculate the maximum frequency deviation. What is the modulation index when the modulation frequency is reduced to 250 Hz and the modulating voltage is raised to 3.2V?														    (5)

24.	a.	State the reason why Super Heterodyne Receiver is used in practical applications when 	compared to TRF.															    (5)

b.	With the neat block diagram explain the operation of Super Heterodyne Receiver.	  (10)

(OR)

25.	a.	Describe the functional characteristics of an uplink, a transponder and a downlink model 	for a satellite system.															  (10)

b.	List the advantages and disadvantages of Geosynchronous Satellites.				    (5)