Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	LINEAR INTEGRATED CIRCUITS AND APPLICATIONS

Time : 3 hours

Subject Code:	09EC220										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is Ion Implatation?

2.	What are the popular IC packages available?

3.	The Bandwidth (BW) is __________ for an ideal operational Amplifier.

4.	Give the unit of slew rate.

5.	List the types of comparators.

6.	The Astable Multivibrator is also called as ____________ Generator.

7.	The roll-off rate ___________ with the order of the filter.

8.	Draw the frequency response of a low-pass filter.

9.	The lock range is usually ___________ than the capture range in PLL.

10.	The Parallel A/D converter is also called as _________ type A/D converter

PART – B  (5 x 3 = 15 MARKS)

11.	What is the significance of using buried layer?

12.	What is the use of differential amplifier? List its types.

13.	List the application of operational amplifier based comparators.

14.	What is a switched capacitor filter?

15.	List the important specifications of A/D and D/A converters.

PART – C  (5 x 15 = 75 MARKS)

16.	With necessary diagrams, briefly explain the fabrications of Complimentary MOSFET.

(OR)

17.	Explain the planar technology of fabricating ICs in detail.

18.	Show with the help of a circuit diagram an op-amp used as:

a.	Multiplier.                                                                                                                       	    (8)

b.	Divider and explain their operation.                                                                              	    (7)

(OR)

19.	Write short notes on the following using op amp:

a.	Logarithmic Amplifier.                                                                                                       (8)

b.	Anti Logarithmic Amplifier                                                                                          	    (7)

20.	Explain the working principle and operation of the Schmitt trigger with necessary circuits and waveforms.

(OR)

21.	Explain the working principle and operation of the Wein’s bridge oscillator with necessary circuits and waveforms.

[P.T.O.]

22.	Draw the functional diagram of IC 555 Timer in monostable mode and explain its operation and derive expression for time period.

(OR)

23.	Design a wide band reject filter having fh  = 400 Hz and fl = 2 kHz having pass band gain as 2.

24.	Explain the operation and perform the closed loop analysis of PLL.

(OR)

25.	With an example and diagram explain the working principle of successive approximation type

ADC.