Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ELECTROMAGNETIC FIELDS							      Time : 3 hours

Subject Code:	09EC221										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Stokes theorem.

2.	Distinguish between scalar quantity and vector quantity.

3.	State Gauss’s law for magnetic field.

4.	Define mmf.

5.	State Poisson’s and Laplace Equation.

6.	Define current density.

7.	Define hysteresis.

8.	State the expression for inductance of a toroid.

9.	Why ∇×E=0?

10.	Compare field theory and circuit theory.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the difference between scalar and vector quantity?

12.	What is the maximum torque on a square loop of 200 turns in a field of flux density ‘B’ Tesla? 	The loop is 15cm wide and carries a current of 5A. What is the magnetic moment of the loop?

13.	Determine the force between two parallel conductors of length 1m separated by 50cm in air and carrying currents of 30A in same direction.

14.	What is torque on a solenoid?

15.	Mention the application of Poynting vector.

PART – C  (5 x 15 = 75 MARKS)

16.	Three equal positive charges of C each are located at three corners of a square, side  10 cm. Determine the magnitude and direction of the electric field at the vacant corner point of the square.

(OR)

17.	Discuss the application of Gauss law and derive appropriate equation its applications.

18.	State Biot-Savart’s law. Obtain the magnetic field intensity H due to an infinite long straight filament carrying current I.

(OR)

19.	In a cable the solid inner conductor of radius ‘a’ carries current I Amperes. The outer annular of inner radius ‘b’ and outer radius ‘c’, carries –I Amperes. Using Amperes circuital law find the expression for magnetic field intensity in all regions.

20.	Derive the boundary conditions between the interface of two dielectric media.

(OR)

21.	The permittivity of the dielectric of a parallel plate capacitor increases uniformly from ϵ1 at the other. If A is the surface are of the plate and  ‘d’ is the thickness of dielectric, derive an expression for capacitance.

[P.T.O.]

22.	An iron ring of relative permeability 200 is wound uniformly with 2 coils of 200 and 610 turns of wire. The cross section of the ring is 10cm2 and the mean circumference is 100cm. Calculate self inductance, mutual inductance and total inductance. When the coil are connected is series and in opposition. .

(OR)

23.	a.	Derive the energy density equation.

b.	Find the energy stored in the solenoid having 75cm long and 8 cm in and is wound with 	1500 turns of wire, carrying a current of 15A.

24.	Derive the Maxwell’s equation of integral form, point form and vector form from faraday’s law and Ampere’s law.

(OR)

25.	State Poynting theorem. Derive the Poynting vector in rectangular coordinates.