Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	COMMUNICATION THEORY AND SYSTEMS			      Time : 3 hours

Subject Code:	09EC222										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define modulation.

2.	Define Bandwidth.

3.	As related to AM, what is over modulation and under modulation?.

4.	Define frequency deviation in FM.

5.	What are the advantages of ring modulator?

6.	Draw the block diagram of Coherent detector.

7.	State the advantage of superheterodyne receiver.

8.	What is Threshold effect in FM?

9.	Define thermal noise.

10.	Define noise factor.

PART – B  (5 x 3 = 15 MARKS)

11.	What is baseband signal?

12.	A 20 MHZ carrier is frequency modulated by a sinusoidal signal such that the peak frequency deviation is 100 KHz. Determine the modulation index and approximate bandwidth of the FM signal if the frequency of the modulating signal is 50 KHz.

13.	Compare low level and high level modulation.

14.	Why pre-emphasis is used in FM system? Sketch a typical pre – emphasis circuit.

15.	Calculate noise figure for a receiver connected to an antenna whose resistance is 100Ω and equivalent noise resistance is 50 Ω .

PART – C  (5 x 15 = 75 MARKS)

16.	What are the types of modulation? Explain in detail the need for modulation.

(OR)

17.	Define communication. Draw and explain the block diagram of a communication system. What is the need for wireless communication?

18.	a.	Derive the equation of AM wave . Draw the Modulated wave for various modulation 	indices.                                                                                        						  (12)

b.  	Compare the performance characteristics between DSB-SC and SSB-SC System.	    (3)

(OR)

19.	Derive the expression for frequency modulated signal. Explain what is narrow band and wide band FM using the expression.

20.	a.	Discuss the generation and detection scheme for standard AM.                   			  (10)

b.	Show that an AM can be recovered, irrespective of value of % modulation by using 	synchronous detection technique.                                                                  			    (5)

(OR)

21.	Draw the circuit diagram of Foster- Seeley discriminator and explain its working.

[P.T.O.]

22.	With a neat block diagram explain the operation of AM super heterodyne receiver.

(OR)

23.	With a neat block diagram explain the operation of FM receiver. Explain the significance of pre-emphasis and de-emphasis in FM.

24.	Discuss in detail the noise performance in SSB-SC receivers.

(OR)

25.	Explain in detail the noise performance in FM receivers.