Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	DIGITAL SIGNAL PROCESSING						      Time : 3 hours

Subject Code:	09EC224										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List any two applications of DFT.

2.	Give the number of complex additions and complex multiplications required to compute 16-point DFT using DIT-FFT algorithm.

3.	Mention the disadvantages of Finite Impulse Response filters.

4.	What are the different design techniques available for the Finite Impulse Response filters?

5.	Draw the magnitude response of a Low-pass Elliptic Filter.

6.	What are the advantages of representing digital systems in block diagram form?

7.	List the three common forms that are used to represent the numbers in a digital hardware.

8.	Which realization is less sensitive to the process of quantization?

9.	What is replication?

10.	Write the broad categories of DSP processors.

PART – B  (5 x 3 = 15 MARKS)

11.	An input sequence x(n)={2, 1, 0, 1, 2} is applied to a DSP system having an impulse response h(n)={5, 3, 2, 1}.  Determine the output sequence produced.

12.	State Gibb’s phenomenon.

13.	Convert the given analog filter in to a digital filter whose system function is

H(s) =s+0.2(s+0.2)2+9.

Use the impulse invariant technique. Assume T=1s.

14.	What is the need for quantization and list the common methods of quantization.

15.	Write about basic Wiener filter theory.

PART – C  (5 x 15 = 75 MARKS)

16.	Find the output y(n) of a filter whose impulse response is h(n)= {1, 1, 1} and input signal

x(n) = {3, –1, 0, 1, 3, 2, 0, 1, 2, 1} using

a.	Overlap-save method.                                                                                 			    (7)

b.	Overlap-add method.															    (8)

(OR)

17.	Compute the DFT of the sequence x (n) =2n, where 0≤n≤7 using DIF FFT algorithm.

18.	Determine the filter coefficients h(n) obtained by sampling

Hd(ejω)=e-j(N-1)ω2 for  0≤ω≤π2=0 forπ2≤ω≤π

for N=7.

[P.T.O.]

(OR)

19.	The desired response of a low-pass filter is

Hd(ejω)=e-j3ω for  −π4≤ω≤π4  =0 forπ4<ω≤π

Find H (z) for N=7 using Hamming window.

20.	Design a digital Chebyshev filter to satisfy the constraints:

0.707≤ |H(ejω)| ≤1, 0≤ω≤0.2π, |H(ejω)| ≤ 0.1, 0.5π ≤ ω ≤ π.

Using bilinear transformation and assuming T=1s.

(OR)

21.	a. 	Determine the direct form I and II realizations for a third- order IIR filter whose system 			function is given by:

H(z)=0.28z2+0.319z+0.040.5z3+0.3z2+0.17z−0.2                                       						(3+5)

b. 	Obtain the cascade realization of the system characterized by the transfer function

H(z)=2(z+2)z(z−0.1)(z+0.5)(z+0.4)                                         						    (7)

22.	Consider the transfer function H(z)= H1(z) H2(z) where

H1(z)= 11-0.9z-1andH2(z)= 11-0.8z-1

Draw the product quantization noise model of the system and determine the overall output    	noise power.

(OR)

23.	Determine the dead band of the filter and explain the characteristics of a limit cycle oscillation with respect to the system described by the equation y(n) = 0.95 y(n – 1) + x(n).

24.	Explain about LMS algorithm used for adaptive filtering with neat flow chart. List out the practical limitations of basic LMS.

(OR)

25.	Draw and explain Harvard architecture and pipelining technique.