Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	MICROPROCESSORS AND MICROCONTROLLERS   	     Time : 3 hours

Subject Code:	09EC233										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List the flags available in 8085.

2.	List the instructions that clear the content of the accumulator of 8085.

3.	How many 16-bit registers are in 8086?

4.	What are the modes in which 8086 can operate?

5.	Which peripheral device is called as Programmable Interval Timer?

6.	What are the different modes of operation possible in keyboard section of 8279?

7.	Mention the purpose of SBUF register of 8051.

8.	List the bit addressable registers of 8051.

9.	What is the function of TMOD register?

10.	Mention the purpose of signal conditioning.

PART – B  (5 x 3 = 15 MARKS)

11.	Find the minimum number of address lines required to access 16K Bytes capacity of RAM in 8085.

12.	State the functions of AAM and AAD instruction of 8086.

13.	Brief on various modes of 8253.

14.	What are the differences between a microprocessor and microcontroller?

15.	What does the instruction MOVX A,@DPTR perform?

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	Draw the timing diagram for opcode fetch cycle.									    (6)

b. 	Discuss on the various hardware and software interrupts available in 8085.			    (9)

(OR)

17.	Draw and explain the architecture of 8085.

18.	Explain the different instruction formats for 8086 CALL and RET instructions.

(OR)

19.	Explain in detail the various processor control instructions	.

20.	Draw the block diagram of 8255 programmable peripheral interface and explain the different modes of operation with their control word formats.

(OR)

21.	Draw and explain the functional block diagram of 8253.

22.	Explain the memory organization of 8051 microcontroller	.

(OR)

23.	Develop an 8051 ALP for performing:

a.	Addition of two 8-bit numbers and 												    (8)

b.	Multiplication of two 8-bit numbers.											    (7)

[P.T.O.]

24.	Discuss about the timers and counters in 8051 microcontroller.

(OR)

25.	Discuss the role of the following in microcontrollers:

a.	Watchdog timers.																    (8)

b.	Real time clock.																    (7)