Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ELECTRONICS AND MICROPROCESSORS			      Time : 3 hours

Subject Code:	09EC235										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List the building blocks of a basic power supply.

2.	What are the biasing conditions to operate a transistor in active region?

3.	A thermistor has  a ___________ temperature coefficient of resistance.

4.	What is piezo electric effect?

5.	Convert the given binary code 10110 into gray code.

6.	Write the truth table of Half subtractor.

7.	What is stack pointer?

8.	Write the PSW format of 8085.

9.	List the types of memory.

10.	What is meant by TRAP?

PART – B  (5 x 3 = 15 MARKS)

11.	A half wave rectifier is used to supply 24 V DC to a resistive load of 500Ω and the diode has 	the forward resistance of 50Ω. Calculate the maximum value of the AC voltage required at the 	input.

12.	List out the principle of a Strain gauge.

13.	Draw the logic diagram of a Full adder and write its truth table.

14.	Write an ALP to add two 8 bit numbers.

15.	Differentiate between memory mapped I/O and peripheral mapped I/O.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the circuit diagram of a RC coupled amplifier and explain its working principle with necessary wave forms.

(OR)

17.	Derive an expression for the efficiency for a full wave rectifier. What is ripple factor? What is the ripple factor value for half wave rectifier and full wave rectifier?

18. 	a. 	What is a Displacement Transducer? Also explain in detail with neat illustrations, the 	principle of working of the electrical strain gauges.

b. 	With a neat illustration, explain the working and the characteristics of a phototransistor.

(OR)

19.  With neat block diagrams, briefly explain the working of the following.

a.	CRO.				       b.	Digital multimeter.

20.	a.   	Realize F(x, y, z) = Σ m (2, 3, 4, 5, 7) using basic gates AND, OR, NOT.

b. 	Implement the following Boolean function.

F (A, B, C, D) = Σ m (0, 2, 6, 10, 11, 12, 13) + d (3, 8, 14) using

i.	8:1 multiplexer.

ii.	Decoder.

[P.T.O.]

(OR)

21.	Write a short note on the following:

a.	Master Slave JK Flip Flop.

b.	4 bit asynchronous counter.

22.	Explain the functional blocks of 8085 microprocessor with a neat diagram.

(OR)

23.	a.	Discuss the instruction format and addressing modes of 8085.

b.	Write an assembly language program to add two 16 bit numbers.

24.	Write notes on the interfacing of I/O devices with 8085 by giving examples.

(OR)

25.	Explain in detail about various data transfer modes.