Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	BASIC ELECTRICAL ENGINEERING					      Time : 3 hours

Subject Code:	09EE101										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The unit of inductance is ___________.

2. 	The frequency of DC supply is_____________Hz.

3. 	What is the unit of magnetic flux?

4. 	Why hard steel is suitable to make permanent magnets?

5. 	What is the form factor of a sine wave?

6. 	Write any one disadvantage of hydroelectric power plant.

7. 	The brushes in DC motor are made of ______________.

8. 	Write any one application of single phase Induction motor.

9. 	What is the use of choke in Fluorescent lamp?

10. 	The energy consumption of MI instrument as compared to MC instrument is_____________.

PART – B  (5 x 3 = 15 MARKS)

11. 	State Ohm’s law and limitations of the law.

12. 	Write short notes on mutual induction.

13. 	Give any three advantages of a three phase system.

14. 	Classify DC motors.

15. 	Draw the connection diagram for stair case wiring.

PART – C  (5 x 15 = 75 MARKS)

16. Write short notes on the following:

a.	Voltage and current division rule in Dc circuits. 									    (5)

b. 	Kirchhoff’s Law and source transformation. 										  (10)

(OR)

17. 	Write short notes on the following:

a. 	Circuit elements with phasor diagrams. 											    (7)

b. 	Star delta transformation. 														    (8)

18. 	a. 	Derive the equation of self and mutual inductance. 								  (12)

b. 	Define coupling co-efficient and magnetic flux density.							    (3)

(OR)

19. 	a. 	Compare electric and magnetic circuit. 											    (8)

b. 	A mild-steel ring of mean circumference 400 mm and average cross-sectional area          			500mm2 is uniformly wound using an insulated conductor of 200 turns. Calculate

(i)  	the reluctance of the ring, and (ii) the current required to produce a flux of 800 μWb

within the ring. Take the relative permeability of mild steel as 380.					    (7)

20. 	Discuss the operation of nuclear power plant.

(OR)

21. 	Explain the operation of hydroelectric plant.

[P.T.O.]

22. 	Explain the principle of operation of DC motor.

(OR)

23. 	Explain the working principle and operation of DC generator.

24. 	With a neat diagram explain the construction and principle of a moving iron attraction        	type instrument.

(OR)

25.	Write short notes on:

a. 	Fluorescent lamp wiring. 														    (8)

b. 	Wiring materials and accessories.		 										    (7)