Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	DIGITAL ELECTRONICS								      Time : 3 hours

Subject Code:	09EE202										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by EBCDIC code?

2.	Simply F = x+x’y.

3.	Draw the logic diagram of Half Adder.

4.	What is meant by parity?

5.	Give the excitation table of T flip-flop.

6.	Find the number of flip-flop in mod-10 Johnson counter.

7.	What are the basic models of sequential machines?

8.	What is meant by state table?

9.	What is FPGA?

10.	Draw the circuit of nMOS inverter.

PART – B  (5 x 3 = 15 MARKS)

11.	Express (26.24)8  in decimal number.

12.	Draw the block diagram of 4-bit parallel adder.

13.	Distinguish between asynchronous and synchronous counter.

14.	Define Fan-in, Fan-out and propagation delay.

15.	Distinguish between PAL and PLA.

PART – C  (5 x 15 = 75 MARKS)

16.	Reduce the given Boolean expression using Quine McCluskey method and verify using Karnaugh map, F(A, B, C, D)=∑(0, 1, 2, 3, 4, 6, 8, 10, 12, 14).

(OR)

17.	Obtain F and F' for the given expression using Karnaugh map. F(A,B,C,D)=∑(4,5,6,7,8)+d∑(10,11,12,13) in SOP and POS form.

18.   Design a gray to binary encoder using AND and OR gates.

(OR)

19.  What is a multiplexer? Implement the given logic function using a multiplexer.

F(X, Y, Z)=∑(0, 2, 6, 7)

20.	Explain the working of master slave flip flop with a neat diagram.

(OR)

21.	Explain the working of ring counter with neat diagram.

22. 	A sequential circuit with three D flip flop A, B, C is described by the following flip flop input function .   DA=(BC'+B'C)x+(BC+B'C')x'

DB=A

DC= B

a.	Derive the state table for the circuit.

b.	Draw two state diagrams one for x=0 and the other for x= 1.

[P.T.O.]

(OR)

23.	Design an asynchronous sequential circuit that has two inputs X2 and X1 and one output Z.

The output is to remain 0 as long as X1 is 0.The first change in X2 that occurs while X1 is a 1 will cause Z to be 1. Z is to remain a 1 until X1 returns to 0.

24.	a.	Compare the performance of various logic families.

b.	Explain the working of CMOS NAND and NOR gate.

(OR)

25.	a.	Explain about the working of FPGA.

b.	Implement any digital function with PAL and PLA.