Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ELECTRICAL MACHINES								      Time : 3 hours

Subject Code:	09EI220											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	How will you change the direction of dc motor?

2.	What is the purpose of starter in dc motors?

3.	What are the main parts of transformer?

4.	Why core of the transformers are laminated?

5.	In which type of three phase induction motor external resistance can be added?

6.	DOL Starters are generally used in three phase induction motor upto ___________ kW.

7.	A tachogenerator adopts the principle of   _______________.

8.	After manufacturing is it possible to change the direction of rotation of shaded pole induction motor? Give justification in one sentence.

9.	What are the applications of electric heating?

10.	Mention the types of motor used in traction.

PART – B  (5 x 3 = 15 MARKS)

11.	Enumerate the losses in a dc motor.

12.	Draw the equivalent circuit of the transformer.

13.	Mention the types of Induction motor rotor construction. Differentiate them.

14.	How an ac servomotor differs from induction motor?

15.	List out the methods of Electric Heating.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Derive the torque equation of dc motor. Explain the speed torque characteristics of dc 	shunt and series motors. 								    						  (10)

b.	Explain the Armature Control of dc motor.					      					   (5)

(OR)

17.	Describe in detail the construction of a dc machine with relevant diagrams.

18.	Derive the emf equation of a single phase transformer. Explain the operation of transformer under no-load and loaded conditions.

(OR)

19.	Explain the procedure adapted for No-load test and Short Circuit test on a single phase transformer. Explain how regulation can be predetermined from these tests.

20.	By deriving the torque equation under running condition of three phase induction motor, explain the speed-torque characteristics.

(OR)

21.	a.	Describe V curves for synchronous motor.					      					   (8)

b.	Explain any one method of synchronization of alternator.			      				   (7)

[P.T.O.]

22.	a.	Prove that the single phase induction motor is not self starting with the help of Double

i.	Field Revolving Theory.							       	       					    (8)

ii.	Explain about the construction and operation of shaded pole induction motor.       (7)

(OR)

23.	Explain about the construction and operation of the following motors.		   			(8+7)

a.	Variable reluctance stepper motor

b.	Repulsion induction motor

24.	a.	Explain any one method of electric heating.					       					    (7)

b.	Draw the schematic diagram for welding generator and explain its operation.	           (8)

(OR)

25.	Draw the Block Diagram of the Locomotive Power system with simplified converter- Inverter power circuits and hence explain the same.