Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	DIGITAL PRINCIPLES AND SYSTEM DESIGN			      Time : 3 hours

Subject Code:	09IT204											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Convert (756)H  to binary.

2.	Write the truth table of Exclusive OR gate.

3.	Draw the block diagram of Combination circuit.

4.	What are the methods available in HDL?

5.	Draw the general structure of decoder.

6.	Define PLDs.

7.	What is meant by Race around condition?

8.	How many flip-flops are needed to build an 8 bit shift register?

9.	What is meant by cycles?

10.	What is static hazard?

PART – B  (5 x 3 = 15 MARKS)

11.	Simplify C(B+C)(A+B+C).

12.	Explain the logical implementation of half adder.

13.	Write short note on EPROM and EEPROM.

14.	Compare Mealy and Moore Machines.

15.	What are the steps for the analysis and design of asynchronous sequential circuit?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Simplify and implement  circuit for the Boolean function

F(A, B, C, D) = ∑ m(2, 3, 5, 7, 11, 12, 13, 14, 15)

(OR)

17.	a.	What is non weighted code and explain each with examples. 						  (10)

b.	State and prove De Morgan’s law. 												    (5)

18.	a.	Design a Full subtractor using NOR gates only.									  (10)

b.	Explain the design procedure for HDL. 											    (5)

(OR)

19.	Explain the design of 8421 BCD to excess 3 code converter.

20.	Explain how you will build a 64 input MUX using nine 8 input MUXs.

(OR)

21.	Draw and explain the block diagram of PLA and implement the functions:

F1(A, B, C) = ∑ m(3, 5, 7)

F2(A, B, C)= ∑ m(4, 5, 7)

22.	Explain serial in serial out and parallel in parallel out shift registers.

(OR)

[P.T.O.]

23.	Using  JK flip flops, design a counter which counts in the sequence:

000, 111, 101, 110, 001, 010, 000….

24.	Explain the classification of Race – Free State Algorithm.

(OR)

25.	Find a circuit that has no static hazards and implement the Boolean function:

F(A, B, C, D) =  ∑ m(0, 2, 6, 7, 8, 10, 12)