Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	MICROPROCESSORS AND INTERFACING			      Time : 3 hours

Subject Code:	09IT206											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name the two internal processing units of 8086/8088.

2.	The _____________ flag is set to indicate that the signed result is out of range.

3.	Identify the addressing mode in the given instruction MOV AL, BL

4.	Which segment register is used to perform the destination address for a string for a string operation?

5.	Is the signal M/IO  an input or output of 8086?

6.	What is a wait state?

7.	Define: Half duplex communication link.

8.	Mode1 of 8254 is known as ______________.

9.	What does STI stand for?

10.	______________ is the other name for a software interrupt.

PART – B  (5 x 3 = 15 MARKS)

11.	Given the segment base value is 1236H and an offset value is 0022H. Calculate the physical address.

12.	Assuming that AX = 0010, BX = 0100, and DS = 1000 what happens if the XLAT instruction is executed?

13.	Describe the operation performed by the instruction IN AX, 1AH  .

14.	Distinguish between synchronous and asynchronous data communication.

15.	What are the key differences between NMI and other external hardware initiated interrupts?

PART – C  (5 x 15 = 75 MARKS)

16.  With a neat diagram explain the internal architecture and system model of 8086/8088.

(OR)

17. 	a.	Describe the function of each status flag in 8086.  				           			    (5)

b.	What is the function of the stack? Discuss push and pop operations with example.      (10)

18. 	Define Addressing mode. Explain various types of addressing modes in 8086/8088 with suitable examples.

(OR)

19. 	a.	Discuss the difference between a JMP and CALL instruction with example.                    (5)

b.	Discuss in detail various data transfer instructions with examples.                                  (10)

20. 	Explain the minimum mode and maximum mode signals in 8086.

(OR)

21. 	a.	What is the duration of the bus cycle in 8088 based computer if the clock is 8MHz and two 	wait states are inserted?                                                                                             	    (5)

b.	Explain in detail the isolated I/O interface.                                                                       (10)

22. 	Explain in detail different modes of operation of 8255A.

[P.T.O.]

(OR)

23. 	Elaborate on the operation of 8279 programmable keyboard/display controller.

24. 	Explain the internal architecture of 8259A and also explain programming with 8259A.

(OR)

25. 	a.	Discuss about internal interrupt functions.                                                                          (8)

b.	Write notes on interrupt vector table and interrupt instructions.                                   	    (7)