Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	SIGNALS AND SYSTEMS								      Time : 3 hours

Subject Code:	09IT207											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	r(t –2) is a

a.	energy signal							b.	power signal

c.	neither energy nor power				d.	none

2.	The system y(t) = x(t) + 2x(t + 3) is

a.	causal system						b.	non-causal system

c.	partly (a) and partly (b)				d.	none of these

3.	A continuous time system with impulse response h(t) = eatu(t) is stable and causal when

a.	stable for a < 0						b.	causal for all a							c.	stable for all a and causal for a = 0		d.	stable for a = 5 and causal for a < 0

4.	The value of Integral −∞∞δ(t)sintdtis equal to

a.	zero			b.	1		c.	infinite		d.	undefined

5.	Magnitude Plot of a Composite signal x(t) = e2jt + e3jt is

a.	half wave rectified sinusoidal			b.	full wave rectified sinusoidal

c.	exponentially increasing sinusoidal		d.	exponentially decreasing sinusoidal

6.	An RLC series circuit has a variable inductance. The value of L for resonance conditions at fundamental frequency is 0.18 H. For resonance conditions at third harmonic frequency the value of inductance is

a.	0.54 H		b.	0.06 H		c.	0.02 H		d.	1.62 H

7.	The analog signal m(t) is given below m(t) = 4cos100πt + 8sin200 πt +cos 300πt, the Nyquist sampling rate will be

a.	1/100		b.	1/200		c.	1/300		d.	1/600

8.	In Laplace transform, multiplication by e-at in time domain becomes

a.	translation by a in s domain			b.	translation by (-a) in s domain

c.	multiplication by e-as in s domain		d.	none of the above

9.	The z transform of sequence x[n] = {2, 4, 3, 2}

a.	2z-1 + 4z-2 + 3 + 2z+1					b.	2z-1 + 3 + 4z+1 + + 2z+2

c.	2z+1 + 3z-1 + 4z-2 + 2z-3				d.	3 + 2z-1 + 4z-2 + 2z-1

10.	The ROC of sequence in the Z.T. of sequence x[n] = an u [n] is

a.	z > a		b.	z < a	c.	|z| > a	d.	|z| < a

PART – B  (5 x 3 = 15 MARKS)

11.	Sketch and label the even and odd components of the signals shown in figure.

[P.T.O]

12.	List the properties of convolution integral.

13.	The transfer function of a system is given by H(ω) =K, where K is a constant. Sketch the magnitude and phase function of this transfer function. Evaluate the impulse response of this filter. Sketch this response and state whether the filter is physically realizable.

14.	Find the initial and final values, if exist, of the signals with Laplace transform given below.

s+5 / (s2+3s+2)

15.	Find the Z transform of the following function and plot their ROC.

x(t) = a2n+1 u[n+3]

PART – C  (5 x 15 = 75 MARKS)

16.	The continuous time signal x(t) is shown in the figure. Sketch the following waveforms

a.	x( t ) u( 1 t )							b.	x( t / 3)

c.	4 x( t ) [ u ( t  2 ) u (t1) ] 				d.	x( t ) δ ( t )

e.	x(5t)

(OR)

17.	Consider a discrete-time system characterized by the following input-output relationship 		          y[n]  = 2x[n+2] – x[n–2]

Determine whether the system is

a.	Linear				b.	Causal		c.	Time-invariant		d.	Memoryless		e.	Stable

18.	Find the convolution of the two continuous time signal given below:

x1(t) = (t + 2t2){u(t + 1) – u(t  –  1)};   	x2(t) = 2u(t + 2)

(OR)

19.	a.	Determine the convolution of the signal x[n] = cos(πn)u[n] with  h[n] = (½)n u[n]. 	(9)

b.	Determine the impulse response for a cascade of two linear time-invariant systems having impulse responses :

h1[n] = (½)n u[n]    and     h2[n] = (¼)n u[n] 										(6)

20.	Consider a casual LTI system implemented a the RL circuit shown in the figure. A current source produces an input current x(t) and the system output is considered to be the current y(t) flowing through the inductor.

a.	Find the differential equation relating x(t) and y(t).

b.	Determine the frequency response of the system by considering the output of the system to inputs of the form x(t) = e jωt

c.	Determine the output y(t) if x(t)= cos(t).

(OR)

21.	a.	Consider a stable LTI system that is characterized by the differential equation

y′′ (t) + 4 y′ (t) + 3 y(t) = x′ (t) + 2 x(t)

Find its response for the input x(t)= e –t u(t).				          					    (9)

b.	The impulse response of a CT system is expressed as:

h(t)={e –t/RC  / (RC)} u(t)

Find the frequency response of the system. Plot the frequency response.    			    (6)

22.	Analyze different techniques incorporated for reconstruction of CT signal from its samples.

(OR)

23.	a.	The input x(t) = e-2t u(t) is given to the system. The output response of the system to the 	input is y(t) = e-t u(t). Find the frequency and impulse response of the system using 	Laplace Transform.															    (9)

b.	Find the inverse Laplace Transform of

X(s) = (2 + 2s e-2s+4 e-2s) / (s2 + 4s + 3 )         Re(s) > –1      						    (6)

24.	a.	Realize the transfer function H(z) = {2z(z+2)} / {(z–0.1)(z+0.5)(z+0.4)} using the direct 	form-I structure.																    (7)

b.	Find the inverse Z-transform of 												    (8)

X(Z) = log (1 – ½ Z-1) ; ROC |Z| > ½

(OR)

25.	Solve the following difference equation using z-transforms:

y[n] +3y[n–1] + 2y[n–2] = 2x[n] – x[n–1]; y[–1]= 0; y[–2] = 1, x[n] = u[n]