Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	COMMUNICATION ENGINEERING					      Time : 3 hours

Subject Code:	09IT232											      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The total power is 1200 W, and the carrier alone is 850 W.  What is the modulation index?

2.	Why is superposition useful?

3.	A signal must slew from –10 to +10 V in 50 μs.  What is the slew rate?

4.	Mention the three major layers of ionosphere.

5.	What is the dynamic range of an 8 bit conversion?

6.	The grouping of eight bits constitutes a _________.

7.	Mention the techniques to improve the performance of modem?

8.	What is null modem?

9.	Mention the downlink frequency of GSM.

10.	Expand POCSAG.

PART – B  (5 x 3 = 15 MARKS)

11.	Discuss about commercial FM broadcast band allocations and side bands.

12.	Mention the roll of Marconi antenna for impedance – matching techniques.

13.	Define μ – law companding with equation and curve.

14.	What are the four groups of information present in the detected signal of a TV receiver?

15.	List out the attractiveness of CDMA.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain superheterodyne AM receiver.											    (8)

b.	Discuss the function of AGC.													    (7)

(OR)

17.	a.	What is the modulation index of NBFM when the modulating signal varies from 300 to 	3000 Hz and the user has 12 KHz of spectrum ±6kHz deviation?					    (3)

b.	With a block diagram and using multiplier explain direct and indirect method of FM?

(12)

18.	a.	Explain the functions of line drivers and receivers.								  (10)

b.	What is CMRR?	How is it measured?											    (5)

(OR)

19.	a.	For the 50 – ft tower, what is the distance if the receiving antenna is at ground level?  What 	if it is on a 100-ft tower?														    (3)

b.	Explain the basic dipole antenna with its radiation pattern.							  (12)

20.	a.	Discuss about network analyzers.												    (5)

b.	With a block diagram explain pulse code modulation.								  (10)

(OR)

21.	a.	Discuss about frame synchronization.											    (5)

b.	With a block diagram and wave form illustration explain delta modulation technique.

(10)

[P.T.O.]

22.	a.	Draw and explain the basic design of CRT.										    (5)

b.	With a block diagram explain the operation of color TV transmitter.				  (10)

(OR)

23.	a.	Draw the functional blocks of a modem and explain each block.					    (7)

b.	Explain about high – speed POTs links using xDSL.								    (8)

24.	a.	Explain a typical transponder with block diagram.								    (5)

b.	Discuss about the control, space and user segments in a GPS system.				  (10)

(OR)

25.	a.	Explain typical digital cellular phone with a neat block diagram.					    (7)

b.	Discuss about cellular system protocol and testing.								    (8)