Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ENGINEERING THERMODYNAMICS					      Time : 3 hours

Subject Code:	09ME202										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a quasi-static process?

2.	When a gas is to be stored, the type of compression that would be ideal is ____________.

3.	A frictionless heat engine can be 100% efficient only if its exhaust temperature is  __________.

4.	____________ is a function of a quantity of heat which shows the possibility of conversion of that heat into work.

5.	With the increase in pressure boiling point of water __________ and enthalpy of evaporation ____________.

6.	The specific volume of water when heated at 0°C first ___________ and then __________.

7.	What is compressibility factor?

8.	What is calorific value?

9.	For same compression ratio thermal efficiency of Otto cycle is ___________ than that of Diesel cycle.

10.	What are influencing factors the work ratio of closed cycle gas turbine plant?

PART – B  (5 x 3 = 15 MARKS)

11.	What do you mean by “Perpetual motion machine of first kind-PMM 1”?

12.	State : Clausius and Kelvin-Planck statement.

13.	Define: Super Heat, Triple point and Critical point.

14.	Write the limitations of Van der Waals’ Equation.

15.	What are Operating Variables affecting on Thermal Efficiency of actual open cycle gas turbine plant?

PART – C  (5 x 15 = 75 MARKS)

16.	A gas contained in a cylinder is compressed from 1 MPa and 0.05 m3 to 2 MPa. Compression is governed by pV1.4 = constant. Internal energy of gas is given by; U = 7.5 pV – 425 kJ, where p is pressure in kPa and V is volume in m3. Determine heat, work and change in internal energy assuming compression process to be quasi-static. Also find out work interaction, if the 180 kJ of heat is transferred to system between same states. Also explain, why is it different from above.

(OR)

17.	A stream of gases at 7.5 bar, 750°C and 140 m/s is passed through a turbine of a jet engine. The stream comes out of the turbine at 2.0 bar, 550°C and 280 m/s. The process may be assumed adiabatic. The enthalpies of gas at the entry and exit of the turbine are 950 kJ/kg and 650 kJ/kg of gas respectively. Determine the capacity of the turbine if the gas flow is 5 kg/s.

18.	An ice plant working on a reversed Carnot cycle heat pump produces 15 tons of ice per day. The ice is formed from water at 0°C and the formed ice is maintained at 0°C. The heat is rejected to the atmosphere at 25°C. The heat pump used to run the ice plant is coupled to a Carnot engine which absorbs heat from a source which is maintained at 220°C by burning liquid fuel of 44500 kJ/kg calorific value and rejects the heat to the atmosphere. Determine :

[P.T.O.]

a.	Power developed by the engine;

b.	Fuel consumed per hour.

Take enthalpy of fusion of ice = 334.5 kJ/kg.

(OR)

19.	An insulated vessel of 0.5 m3 capacity is divided by a rigid conducting diaphragm into two chambers A and B, each having a capacity of 0.25 m3. Chamber A contains air at 1.4 bar pressure and 290 K temperature and the corresponding parameters for air in chamber B are 4.2 bar and 440 K. Calculate :

a.	Final equilibrium temperature,

b.	Final pressure on each side of the diaphragm, and

c.	Entropy change of system.

For air take cv =  0.715 kJ/kg K and R = 0.287 kJ/kg K.

20.	A pressure cooker contains 1.5 kg of saturated steam at 5 bar. Find the quantity of heat which must be rejected so as to reduce the quality to 60% dry. Determine the pressure and temperature of the steam at the new state.

(OR)

21.	A piston-cylinder contains 3 kg of wet steam at 1.4 bar. The initial volume is 2.25 m3. The steam is heated until its temperature reaches 400°C. The piston is free to move up or down unless it reaches the stops at the top. When the piston is up against the stops the cylinder volume is 4.65 m3.  Determine the amount of work and heat transfer to or from steam.

22.	One kg-mol of oxygen undergoes a reversible non-flow isothermal compression and the volume decreases from 0.2 m3/kg to 0.08 m3/kg and the initial temperature is 60°C. If the gas obeys Van der Waals’ equation find :

a.	The work done during the process 	b.	The final pressure.

(OR)

23.	The following particulars refer to an experimental determination of the calorific value of a sample of coal containing 88% C and 4.2% H2. Weight of coal = 0.848 gm, weight of fuse wire 0.027 gm, of calorific value 6700 J/gm, weight of water in the calorimeter = 1950  gm, water equivalent of calorimeter = 380 gm, observed temperature rise = 3.06°C, cooling correction = + 0.017°C. Find the higher and lower calorific values of the coal.

24.	The stroke and cylinder diameter of a compression ignition engine are 250 mm and 150 mm respectively. If the clearance volume is 0.0004 m3 and fuel injection takes place at constant pressure for 5 per cent of the stroke determine the efficiency of the engine. Assume the engine working on the diesel cycle.

(OR)

25.	Find the required air-fuel ratio in a gas turbine whose turbine and compressor efficiencies are 85% and 80%, respectively. Maximum cycle temperature is 875°C. The working fluid can be taken as air (cp = 1.0 kJ/kg K,  γ = 1.4) which enters the compressor at 1 bar and 27°C. The pressure ratio is 4. The fuel used has calorific value of 42000 kJ/kg. There is a loss of 10% of calorific value in the combustion chamber.