Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	THERMAL ENGINEERING							      Time : 3 hours

Subject Code:	09ME203					      		       			       Maximum Marks: 100

#### Answer ALL questions

(Use of Standard Thermodynamic Tables, Mollier Diagram, Psychometric Chart and Refrigerant Property Tables are permitted)

PART – A  (10 x 1 = 10 MARKS)

1.	Name any two component names of the I.C engine.

2.	How many revolutions will takes place for to complete one cycle for the two stroke engine?

3.	What are the types of nozzles?

4.	Define impulse turbine.

5.	Define discharge pressure of the compressor.

6.	What is the purpose of inter-cooler in the air compressor?

7.	Name any two refrigerants used in vapour compression refrigeration cycle.

8.	Specify any two properties of a refrigerant.

9.	What is meant by absolute humidity?

10.	Name any two factors which affect comfort air conditioning.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the factors which are considered for detonation?

12.	Differentiate impulse turbine and reaction turbine.

13.	Differentiate reciprocating and rotary air compressor.

14.	What is the function of the expansion valve?

15.	What is meant by year-round air-condition system?

PART – C  (5 x 15 = 75 MARKS)

16.	Compare the two-stroke and four-stroke engine with its neat sketch.

(OR)

17.	Explain with the neat sketch of Magneto Ignition system.

18.	Explain the velocity compounding of an impulse turbine with diagram.

(OR)

19.	The velocity of steam at inlet to a simple impulse turbine is 1000m/s and the nozzle angle is 200. The mean blade speed is 400m/s and the blades are symmetrical. The mass flow rate of steam is 0.75 kg/s. The friction effects on the blades are negligible. Estimate:

a.	the bade angles	b.	the tangential force on the blades	c.	the axial thrust

d.	the diagram power 	e.	the diagram efficiency.

20.	Draw and explain the two-stage reciprocating air compressor with intercooler.

(OR)

21.	A two stage compressor takes in 2.82m3 of air per minute at a pressure of 1.05 bar and temperature of 220 C. It delivers the air at 8.44 bar. The compression is carried out in each cylinder according to the law pv1.2= constant. The air cooled to its initial temperature in intercooler. Neglecting clearance, find the minimum power required to drive the compressor.

[P.T.O]

22.	Draw and explain the mechanism of a simple vapour compression refrigeration system.

(OR)

23.	A vapour compression refrigeration plant works between pressure limits of 5.3 bar and 2.1 bar. The vapour is superheated at the end of compression, its temperature being 370 C. The vapour is superheated by 50 C before entering the compressor.

If the specific heat of superheated vapor is 0.63 kJ/kgK, find the coefficient of performance of the plant. Use the data given below:

| Pressure, bar | Saturation temperature, 0 C | Liquid heat, kJ/kg | Latent heat, kJ/kg |
| --- | --- | --- | --- |
| 5.3 | 15.5 | 56.15 | 144.9 |
| 2.1 | 14.0 | 25.12 | 158.7 |

24.	Draw and explain the sensible heating with the derivation of coil efficiency.

(OR)

25.	A small office hall of 25 persons capacity is provided with summer air-conditioning system with the following data:

Outside conditions			-	340 C DBT and 280 C WBT

Inside conditions				-	240 C DBT and 50% RH

Volume of air supplied		-	0.4m3/min/person

Sensible heat load in room		-	125600kJ/h

Latent heat load in the room	-	42000kJ/h

Find the sensible heat factor of the plant.