Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	HEAT AND MASS TRANSFER							      Time : 3 hours

Subject Code:	09ME204										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define thermal conductivity.

2.	Define thermal contact resistance.

3.	In graphical method ____________ is defined as the ratio of number of flowlanes / tubes divided by the number of temperature increments.

4.	Define Biot number.

5.	Write the dimensionless parameters used in forced convection.

6.	Define Fin efficiency.

7.	An automobile radiator is ____________ type of heat exchanger.

8.	List the classification of heat exchangers based on flow direction.

9.	Define emissive power (Eb).

10.	The heat transfer by radiation takes place by means of ____________.

PART – B  (5 x 3 = 15 MARKS)

11.	State Fourier law of heat conduction.

12.	Define Lumped heat analysis.

13.	A horizontal heated plate measuring 1.5 m × 1.1 m and at 215˚C, facing upwards, is placed in still air at 25˚C. Calculate the heat loss by natural convection. The convective film coefficient for free convection is given by the following empirical relation  h = 3.05 (Tf)1/4 W/m2˚C  where, Tf is the mean film temperature in degrees Kelvin.

14.	Define ‘fouling’ in heat exchangers.

15.	A   black  body  at 3000 K  emits  radiation. Calculate:	a.	total emissive power

b.	the total emissive of the furnace if it is assumed as a real surface having emissivity equal to 0.85.

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the general heat conduction equation in cylindrical coordinates.

(OR)

17.	The furnace wall consists of 120 mm wide refractory brick and 120 mm wide insulating fire brick separated by an air gap. The outside wall is covered with a 12 mm thickness of plaster. The inner surface of the wall is at 1090˚C and the room temperature is 20˚C. The heat transfer coefficient from the outside wall surface to the air in the room is 18 W/m2˚C, and the resistance to heat flow of the air gap is 0.16 K/W. If the thermal conductivities of the refractory brick, insulating fire brick, and plaster are 1.6,  0.3 and 0.14 W/mK, respectively calculate:

a.	Rate at which heat is  lost per m2 of the wall surface		b.	Each interface temperature; c.	Temperature of the outside surface of the wall.(Take surface area= 1m2).

18.	Alloy steel ball of 12 mm diameter  heated to 800˚C is quenched in a bath at 100˚C. The material properties of the ball are k = 205 kJ/m hr K, ρ = 7860 kg/m3,  Cp = 0.45 kJ/kg K,

h = 150 kJ/hr m2 K. Determine:	a.	Temperature of ball after 10 seconds 	b.	Time for ball to cool to 400˚C.

[P.T.O.]

(OR)

19.	A pipe 600 mm in diameter carrying steam at a temperature of 180 ο C and another pipe 180 mm diameter carrying water at 12 ο C are buried under the ground with centre to centre distance 1.8 m. The length of each pipe is 90 m and thermal conductivity of soil is 0.45 W/m ο C. Assuming that the pipes run parallel to each other, the earth is infinite and resistance of the pipe material negligible, determine:

a.	the net heat transfer rate between the two pipes  	b.	the rise in water temperature due to heat transfer if the velocity of the water in the pipe is 0.05m/s.

20.	Air at 200C and 1atm pressure is flowing over a flat plate at a velocity of 3m/s. If plate is

280 mm wide and at 560C, Calculate the following quantities at  x =280 mm:

a.	Boundary layer thickness 		b.	Local friction coefficient  		c.	average friction coefficient  		d.	Shearing stress due to friction 		e.	Thermal boundary layer thickness 	f.	Local convective heat transfer coefficient  		g.	Average convective heat transfer coefficient 		h.	Rate of heat transfer by convection 		i.	Total mass flow  rate through the boundary   		j.	Total drag force on the plate.

(OR)

21.	A motor body is 360 mm in diameter outside and 240 mm long. Its surface temperature should not exceed 550C when dissipating 340 W. Longitudinal short fins of 15 mm thickness and 40 mm height are proposed. The convection coefficient is 40 W/m20C. Determine the number of  fins required. Atmospheric temperature is 300C .Thermal conductivity = 40 W/m0C.

22.	a.	Derive an expression for LMTD of a counter-flow heat exchanger.           			    (8)

b.	Water enters a cross flow heat exchanger (both fluids unmixed) at 50C and flows at the rate 	of  4600 kg/hr to cool  4000 kg/hr of air that is initially at 400C. Assume the U value to be 	150 W/m2K. For an exchanger surface area of   25 m2, calculate the exit temperature of air 	and water.                         														    (7)

(OR)

23.	a.	Two large tanks, maintained at the same temperature and pressure are connected by a 	circular 0.15 m diameter direct which is 3 m in length. One tank  contains a uniform 	mixture  of  60 mole %  ammonia and 40 mole  % air and the other tank contains a uniform 	mixture of 20 mole %  ammonia and 80 mole % air. The system is at 273 K  and

1.013 × 105  pa.  Determine  the rate of ammonia transfer between two tanks. Assuming a 	steady state mass transfer                  											    (8)

b.	Air at 1 atm and  250C containing small quantities of iodine flows with a velocity of

6.2 m/s inside a 35 mm diameter tube. Calculate mass transfer coefficient for iodine. The thermophysical properties of air are ʋ = 15.5 × 10-6 m2/s ; D = 0.82 × 10-5 m2/s.              (7)

24.	a.	Water is boiled at a rate of 30 kg/h in a copper pan, 30 cm in diameter, at atmospheric 	pressure. Estimate the temperature of the bottom surface of the pan assuming nucleate 	boiling conditions. (Csf = 0.0130).                             								    (7)

b.	A horizontal tube of outer diameter 2.2 cm is exposed to dry steam at 1000C.The pipe surface is maintained at 620C by circulating water through it.  Calculate the rate of formation of condensate per metre length of the pipe.								    (8)

(OR)

25.	a.	A liquid oxygen is stored in double walled spherical vessel.  Inner wall temperature is  ̶̶  	1600C and outer wall temperature is 300C. Inner diameter of sphere is 20 cm and outer 	diameter is 32 cm. Calculate the following:		i.	Heat transfer if emissivity of 	spherical surface is 0.05. 		ii.	Rate of evaporation of liquid oxygen if its rate of 	vapouization of latent 	heat is 200 kJ/kg.              									    (8)

b.	Two very large parallel plates with emissivities 0.5 exchange heat. Determine the 	percentage reduction in the heat transfer rate if  a polished aluminium radiation shield of

Ɛ = 0.04 is placed in between the plates.                          							    (7)