Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	MECHANICS OF MACHINES - I						      Time : 3 hours

Subject Code:	09ME205										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A combination of kinematic pairs, joined in such a way that the relative motion between the links is completely constrained is called ___________.

2.	The mechanism forms a structure , when number of degrees of freedom is equal to ___________.

3.	The power transmitted by a belt is maximum when the maximum tension in the belt (T) is equal to ___________Tc.

4.	The frictional torque transmitted in a conical pivot bearing considering uniform wear is ___________.

5.	Offset is provided in cam follower mechanism to ___________.

6.	The angle between the direction of the follower and normal to the pitch curve is called ___________.

7.	The module is the reciprocal of ___________.

8.	In a simple gear train , if the number of idle gear is odd , then the motion of the driven gear will be ___________.

9.	The height of a Watt’s governor is equal to ___________.

10.	In Hartnell governor , if a spring of greater stiffness is used , then the governor will be ___________.

PART – B  (5 x 3 = 15 MARKS)

11.	Define degrees of freedom.

12.	Define co-efficient of friction.

13.	Define pitch curve of a cam.

14.	State the law of gearing.

15.	What is meant by equilibrium speed of a governor?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain any three inversions of a single slider crank chain mechanism.

(OR)

17.	In a slider crank mechanism the crank is 200 mm long and rotates at 40rad/sec in a CCW direction. The length of the connecting rod is 800 mm. When the crank turns through 60o from Inner-dead centre, determine,

a.	The velocity of the slider.

b.	Velocity of point E located at a distance of 200 mm on the connecting rod extended.

c.	The position and velocity of point F on the connecting rod having the least absolute 	velocity.

d.	The angular velocity of connecting rod.

[P.T.O.]

18.	A shaft rotating at 20rpm drives another shaft at 300 rpm and transmits 6 kW through a belt. The belt is 100 mm wide and 10 mm thickness. The distance between the shafts is 4m. The smaller pulley is 0.5m in diameter. Calculate the stress in the belt, if it is an open belt drive.

(OR)

19.	A thrust shaft of a ship has 6 collars of 600 mm external diameter and 300 mm internal diameter. The total thrust from propeller is 100 kN. If the coefficient of friction is 0.12 and speed of the engine90 rpm, find the power absorbed in friction at the thrust block, assuming 1.uniform pressure  and  2. Uniform wear.

20.	Draw the cam profile for following conditions:

Follower type = roller follower, in-line; lift = 25 mm; base circle radius = 20 mm; roller radius = 5mm; out stroke with UARM, for 1200 cam rotation; dwell for 600 cam rotation; return stroke with UARM, for 900 cam rotation; dwell for the remaining period. Determine max. velocity and acceleration during out stroke and return stroke if the cam rotates at 1200 rpm in clockwise direction.

(OR)

21.	Draw the cam profile for following conditions:

Follower type = flat faced follower, in line; follower rises by 20 mm with SHM in 1200 of cam rotation, dwells for 300 of cam rotation; returns with SHM in 1200 of cam rotation and dwells during the remaining period. Base circle radius = 25mm.

22.	Derive the expression for length of path of contact of a spur gear.

(OR)

23.	In an epicyclic gear train shown in figure, the arm A is fixed to the shaft S. The wheel B having 100 teeth rotates freely on the shaft S. The wheel F having 150 teeth driven separately. If the arm rotates at 200 rpm and wheel F at 100 rpm in the same direction; find:

a.	number of teeth on the gear C 		b.	speed of wheel B.

24.	The arms of a Porter governor are each 250 mm long and pivoted on the governor axis. The mass of each ball is 5kg and mass of central sleeve is 30 kg. The radius of rotation of balls is 150 mm when sleeve begins to raise and reaches the value of 200 mm for maximum speed. Determine the speed range of the governor. If the friction at the sleeve is equivalent of 20N of load at the sleeve, determine how the speed range is modified.

(OR)

25.	The mass of the turbine rotor of a ship is 20 tonnes and has a radius of gyration of 0.6m. its speed is 200 rpm. The ship pitches 60 below and 60 above the horizontal position. A complete oscillation takes 30 seconds and the motion is simple harmonic. Determine the	 .
a. 	Maximum gyroscopic couple,

b. 	Maximum angular acceleration of the ship during pitching, and

c. 	The direction in which the bow will tend to turn when rising, if the rotation of the rotor is 	clockwise when looking from the left.