Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	DESIGN OF MACHINE ELEMENTS					      Time : 4 hours

Subject Code:	09ME206										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Failure of a material is called fatigue when it fails____________.

2.	Stress concentration factor is defined as the ratio of____________________.

3.	When helical compression spring is cut into halves, the stiffness of the resulting spring will be___________.

4.	The resistance to fatigue of a material is measured by_______________.

5.	Two shafts will have equal strength, if____________.

6.	The type of stresses developed in the key _______________.

7.	The diameter of the rivet hole is usually __________ the nominal diameter of the rivet.

8.	A cotter joint is used to transmit_____________.

9.	A connecting rod is designed as a_______________.

10.	The cross-section of the flywheel arms is usually____________.

PART – B  (5 x 3 = 15 MARKS)

11.	Why factor of safety is important in design?

12.	Explain fatigue limit.

13.	What is critical speed?

14.	Mention the various forms of pipe joints.

15.	Define Coefficient of Fluctuation of Energy.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Discuss the effect of surface finish and size factor on endurance limit. 				(10)

b.	Mention the factors to be considered while designing machine.			 			(5)

(OR)

17.	Explain the principal theories of failure for a member subjected to bi-axial stress.

18.	Design a helical spring for a spring loaded safety valve (Ramsbottom safety valve) for the following conditions: Diameter of valve seat = 65 mm; Operating pressure = 0.7N/mm2; Maximum pressure when the valve blows off freely = 0.75N/mm2; Maximum lift of the valve when the pressure rises from 0.7 to0.75 N/mm2 = 3.5 mm ; Maximum allowable stress = 550 MPa ;Modulus of rigidity = 84 kN/mm2; Spring index = 6. Draw a neat sketch of the free spring showing the main dimensions.

(OR)

19.	A machine component is subjected to a flexural stress which fluctuates between + 300 MN/m2 and – 150 MN/m2. Determine the value of minimum ultimate strength according to

a. 	Gerber relation; 		b. 	Modified Goodman relation 	c. 	Soderberg relation. Take yield strength = 0.55 Ultimate strength; Endurance strength = 0.5 Ultimate strength; and factor of safety = 2.

[P.T.O.]

20. 	Compare the weight, strength and stiffness of a hollow shaft of the same external diameter as that of solid shaft. The inside diameter of the hollow shaft being half the external diameter. Both the shafts have the same material and length.

(OR)

21. 	Design a clamp coupling to transmit 30 kW at 100 r.p.m. The allowable shear stress for the shaft and key is 40 MPa and the number of bolts connecting the two halves are six. The permissible tensile stress for the bolts is 70 MPa. The coefficient of friction between the muff and the shaft surface may be taken as 0.3.

22.	Design a knuckle joint for a tie rod of a circular section to sustain a maximum pull of 70 kN. The ultimate strength of the material of the rod against tearing is 420 MPa. The ultimate tensile and shearing strength of the pin material are 510 MPa and 396 MPa respectively. Determine the tie rod section and pin section. Take factor of safety = 6.

(OR)

23. 	Find the efficiency of the following riveted joints:

a. 	Single riveted lap joint of 6 mm plates with 20 mm diameter rivets having a pitch of         50 mm.

b. 	Double riveted lap joint of 6 mm plates with 20 mm diameter rivets having a pitch of        65 mm.

Assume	Permissible tensile stress in plate = 120 Mpa.

Permissible shearing stress in rivets = 90 Mpa.

Permissible crushing stress in rivets = 180 Mpa.

24. 	Design a cast iron piston for a single acting four stroke engine for the following data: Cylinder bore = 100 mm ; Stroke = 125 mm ; Maximum gas pressure = 5 N/mm2 ; Indicated mean effective pressure = 0.75 N/mm2 ; Mechanical efficiency = 80% ; Fuel consumption = 0.15 kgper brake power per hour ; Higher calorific value of fuel = 42 × 103 kJ/kg ; Speed = 2000 r.p.m. Any other data required for the design may be assumed.

(OR)

25.	Derive the expression of the energy stored in a flywheel.