Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	GAS DYNAMICS AND JET PROPULSION				      Time : 3 hours

Subject Code:	09ME207							      		       	       Maximum Marks: 100

#### Answer ALL questions

(Use of Gas Tables is permitted)

PART – A  (10 x 1 = 10 MARKS)

1.	Define critical velocity of fluid.

2.	What is the value of hypersonic flow?

3.	Write down the equation for impulse function

4.	Convergent section acts as a diffuser under ________ sonic condition and acts as nozzle under ________ sonic condition.

5.	Give the two examples for fanno flow.

6.	Define fanning’s coefficient of skin friction.

7.	What is normal shock?

8.	Give two applications of moving shock wave.

9.	Define thrust.

10.	Name the two fuel of bipropellant.

PART – B  (5 x 3 = 15 MARKS)

11.	A plane travels at a speed of 2100km/h in an atmosphere of 280K. Find the Mach angle.

12.	What is chocked flow through a nozzle?

13.	Differentiate Fanno flow and Rayleigh flow.

14.	Calculate the strength of shock wave when normal shock appears at M=4.

15.	What is thrust augmentation?

PART – C  (5 x 15 = 75 MARKS)

16. 	An aircraft is flying at an altitude of 12,000 meters (T= 216.65 K, p= 0.193 bar) at a Mach number of 0.82. the cross-sectional area of the inlet diffuser before the L.P. compressor stage is 0.5 m2. Determine

a.	The mass of air entering the compressor per second

b.	The speed of the aircraft and

c.	The stagnation pressure and temperature of air at the diffuser entry.

(OR)

17. 	A gas flows in a duct of 40 cm diameter at inlet pressure of 3 bar, temperature of 450 K and a velocity of 180 m/s. Calculate at inlet.

a.	Mass flow rate

b.	Stagnation temperature

c.	Mach number

d.	Stagnation pressure values assuming the flow as compressible and incompressible. Take γ=1.3 and cp=1.05 kJ/kgK

18. 	Air is discharged from a receiver at p0=6.91 bar and T0= 3250 C  through a nozzle to an exit pressure of 0.98 bar. If the flow rate is 3600kg/h. Determine for isentropic flow. A) Area, pressure and velocity at throat, a) Area and Mach number at exit and c) Maximum possible velocity.

[P.T.O]

(OR)

19. 	Air expands isentropically the convergent nozzle from constant inlet conditions p0= 4 bar,     T0= 550 K. Exit area of nozzle is 1000 cm2. Determine the exit velocity and the mass flow rate for the following two cases at exit. 		a.	M=1	b.	M=0.85

20. 	The friction factor for 50mm diameter steel pipe is 0.005. At the inlet to the pipe the velocity is 70 m/s, temperature is 80 º C and the pressure is 10 bar. Find the temperature, pressure and Mach number at exit if the pipe is 25m long. Also determine the maximum possible length.

(OR)

21. 	The Mach number at inlet and exit for a Rayleigh flow are M1=3 and M2=1.5. at inlet static pressure of air is 50 kPa and stagnation temperature is 295 K. Find

a.	All properties at exit

b.	Stagnation pressure

c.	Heat transferred

d.	Maximum possible heat transfer

e.	Change in entropy between section 1 and 2.

22. 	Air flows adiabatically in a pipe. A normal shock wave is formed. The pressure and temperature of air before the shock are 150 kN/m2 and 250 C respectively. The pressure just after the normal shock is 350 kN/m2. Calculate

a.	Mach number before the shock

b.	Mach number, static temperature and velocity of air after the shock wave

c.	Increase in density of air

d.	Loss of stagnation pressure of air

e.	Change in entropy

(OR)

23. 	Derive the Prandtl-Meyer relationship for a normal shock.

24. 	With neat sketches explain the principle of operation of:

a.	Turbofan engine.

b.	Ram jet engine.

(OR)

25. 	Discuss in detail of the various propellants used in solid fuel rockets and liquid fuel systems. Also sketch the propellant feed-system for a liquid propellant rocket motor.