Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	DESIGN OF TRANSMISSION SYSTEMS			      	     Time : 3 hours

Subject Code:	09ME208										      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	__________ is a bearing which carries a load by placing rolling element between the two bearing rings.

2.	The radial distance from the pitch circle of a cogwheel or worm wheel to the crests of the teeth or ridges is known as __________.

3.	When the axes of two shafts are parallel, use __________ gear

4.	The axes intersect and the teeth are curved is known as __________.

5.	Classify the types of bevel gears.

6.	State true or false

“Mitre gears are used for connecting non-intersecting shafts”.

7.	__________ is the commonly used gear tooth profile.

8.	Cam transforms __________ motion into __________ motion.

9.	__________ Mechanism does not interfere while raising the load, but prevents the load coming down due to gravity.

10.	A Geneva wheel has 6 slots. Driving crank radius is 50 mm. The distance between the centre is __________.

PART – B  (5 x 3 = 15 MARKS)

11. 	List the advantages of sliding contact over roller bearing.

12. 	Write the advantages and disadvantage of helical gear.

13. 	What are the commonly used materials in worm and wheel?

14. 	What is meant by step ratio?

15. 	Define Pressure angle.

PART – C  (5 x 15 = 75 MARKS)

16. 	Design a suitable Journal bearing for a centrifugal pump from the following available data. Load on the bearing is 13.25 KN. Diameter of the journal = 80 mm, RPM =1440, Bearing Characteristic number =30 x10 -6, Pressure= 0.7 to 1.4 N/mm2, Average temperature = 30oC.

(OR)

17. 	A V-belt drive is required to transmit 16 kW power to a compressor. The motor speed is      1440 rpm and the speed reduction ratio is 3.6. Design the belt drive. Sketch the details of pulley.

18. 	A spur gear pair is to transmit 1.5 kW at 1440 rpm from an electric motor to an air compressor running at 720 rpm. Material to be used is Cast Iron grade 25 for both pinion and wheel. Design a gear and draw a neat sketch

(OR)

19. 	A pair of helical gears is to transmit 30 kW power at a speed reduction ratio of 4:1. The input shaft rotates at 2000 rpm. Design the gears by  assuming a life of 12,000 hours.

[P.T.O.]

20. 	Two straight bevel gears are used in a speed reducer with a transmission ratio of 2. The wheel is supported on both sides and the pinion is over hanging. The input is from a 20 kW electric motor running at 950 r.p.m. Design the bevel gears.

(OR)

21. 	Design a worm gear drive to transmit 22.5 kW at a worm speed of 1440 rpm. Velocity ratio is 24:1. An efficiency of at least 85% is desired. The temperature rise should be restricted to      40° C.

22. 	Design the layout of a 12 speed gear box for a lathe. The minimum and maximum speeds are 100 and 1200 rpm. Power is 5 KW from 1440 RPM induction motor. Construct the speed diagram using a standard speed ratio. Calculate the number of teeth in each gear wheel and sketch the arrangement of the gear box.

(OR)

23. 	Design a 9 speed gear box for the following data:

Minimum speed: 100 rpm, Step ratio: 1.25. The input is from a 4 kW, 1400 rpm motor. Draw the speed diagram and indicate the number of teeth on each gear in a kinematic diagram.

24. 	Design a ratchet wheel for a winch. Data given: Braking torque = 150 Nm: Brake drum diameter = 350 mm.

(OR)

25. 	Draw the displacement-time, velocity-time and acceleration-time curves for a follower in order to satisfy the following condition:

Stroke of the follower 25 mm

Outstroke takes place with SHM during 900 of cam rotation

Return stroke takes place with SHM during 750 of cam rotation

Cam rotates with a uniform speed of 800 rpm.

Also determine the induced contact stress, assuming follower mass as 0.25 kg and spring stiffness as 100 N/mm, when the cam rotation is 00 and 150. Take the thickness of the plate cam as 10 mm. Rb = 25 mm, Rr = 10 mm. Material used is steel.