Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	HEAT ENGINES AND FLUID MACHINERY			      Time : 3 hours

Subject Code:	09ME228										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define the term: surface tension.

2. 	What is the use of air vessel in a reciprocating pump?

3. 	Why we need a draft tube in a reaction turbine?

4. 	Define specific speed of a turbine.

5. 	State first law of thermodynamics.

6. 	State Kelvin-Planck statement.

7. 	What is an air standard cycle?

8. 	Define air standard efficiency.

9. 	State Fourier’s law of heat conduction.

10. 	Define shape factor based on radiation heat transfer.

PART – B  (5 x 3 = 15 MARKS)

11. 	Compare centrifugal pump with reciprocating pump.

12. 	Why turbines are to be governed? Why we need surge tanks in a piping system?

13. 	State Carnot theorem for heat engines.

14. 	What is a heat balance sheet?

15. 	State any two laws of radiation heat transfer.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	One end of the long rod is inserted into a furnace with the other end projecting into the 	ambient air outside. After steady state is reached, the temperature of the rod is measured at 	two points 10 cm apart are found to be 125C and 91C when the ambient temperature is 	28 C. If the rod is 2cms in diameter and h = 15 W/m2K, what is the thermal conductivity of 	the rod? 																		    (8)

b.	Determine the heat loss per hour from a wall of a building when the wind is blowing parallel to its surface with a speed of 2km/h. The wall is 5m long and 3m high. Temperature of the wall is 25C and air temperature is 5C. 							    (7)

(OR)

17.	a.	A meat slab of 25mm thick having K = 1W/mK is heated with the help of microwave 		heating for roasting the meat slab. The centre temperature of the slab is maintained at      	100 	C when surrounding temperature is 30 C, heat transfer coefficient on the surface of the 	meat slab is 20W/m2C. Find out the microwave heating capacity in W/m3.			    (6)

b.	An insulated steam pipe having outside diameter of 3cms is to be covered with two layers of insulation each having a thickness of 2.5cms. The average thermal conductivity of one material is 5 times that of the other. Assuming that the inner and outer surface temperatures of composite insulation are fixed, how much will the heat transfer be reduced when the better insulating material is next to the pipe than it is outer layer?					    (9)

[P.T.O.]

18. 	Explain the following properties of fluid: Surface tension, Viscosity, Compressibility, specific weight and Capillarity.

(OR)

19. 	The outer diameter of   an impeller of a centrifugal pump is 40 cm and the outlet width is 5 cm . The pump is running at 800 r.p.m. and is working against a total head of 15 m.The vane angle at outlet is 40 and the manometric efficiency is 75%, determine:

a.	velocity of flow at outlet		b.	velocity of water leaving the vane		c.	angle made by the absolute velocity at outlet with the direction of motion at outlet 	d.	discharge. Draw the velocity triangle for centrifugal pump.									     (5 x 3)

20. 	A Pelton wheel develops 8000kW under a net head of 130 m at a speed of 200 r.p.m. Assuming the coefficient of velocity for the nozzle as 0.98, hydraulic efficiency of 87%, speed ratio of 0.46 and jet diameter to wheel diameter ratio is 1/9, determine:

a.	discharge required	b.	diameter of wheel	c.	diameter of jet and number of jets

d.	specific speed. Assume mechanical efficiency is 75%.

(OR)

21. 	A Francis turbine with an overall efficiency of 76% is required to produce 150 kW.itis working under a head of 8m.the peripheral velocity =0.25(2gH)0.5 and the radial velocity of flow at inlet is 0.95 (2gH)0.5. The wheel runs at 150 r.p.m.  and the hydraulic losses in the turbine are 20% of the available energy. Assuming radial discharge, determine:

a.	guide blade angle		b.	wheel vane angle at inlet	c.	diameter of wheel at inlet

d.	the width of the wheel at inlet.

22. 	a.	Two streams of air, one at 1 bar, 27 C and velocity 30 m/s   and the other at 2 bar, 227 C 		and 50 m/s velocity mix in equal proportion in a chamber from which heat at the rate of    	50 kJ/kg is removed. The mixture is passed through an adiabatic nozzle .Find the velocity 	of the stream at the nozzle exit. Temperature of the air stream issued by the nozzle is 27 C 	and assume that kinetic energy is conserved.										    (9)

b.	Derive an expression for work done in a polytropic process. 						    (6)

(OR)

23. a.		Two Carnot engines work in series between source and sink temperature of 500K and 	300K.If both engines develop equal power, determine the intermediate temperature.	    (7)

b.		Prove that a system which satisfies Kelvin-Planck statement cannot violate Clausius 	statement. 																	    (8)

24. 	The following data were recorded during a trial on four stroke diesel engine : number of cylinders –4 , diameter and stroke  of the piston = 10 cm and 15 cm respectively, indicated mean effective pressure = 0.67 MPa, speed = 2000 rpm, number of explosion = 980 /min, brake torque = 181.5Nm,fuel consumption =11.89 kg/hr, calorific value of fuel = 41800 kJ/h, relative efficiency on brake power basis =0.5, quantity of jacket cooling water = 1020 kg/h, rise in temperature of cooling water = 35C,specific heat of cooling water = 4.18 kJ/kgK. Determine: air standard efficiency, mechanical efficiency, brake thermal efficiency, brake specific fuel consumption and percent heat loss to jacket cooling water.

(OR)

25.  An ideal diesel cycle operates on 1kg of standard air with an initial pressure of 0.98bar and a temperature of 35C. The pressure at the end of the compression is 33 bar and the cut off is 6% of the stroke. Determine: compression ratio, percentage clearance, heat supplied, heat rejected, thermal efficiency and mean effective pressure.