Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ELECTRON DEVICES									      Time : 3 hours

Subject Code:	10EC201										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define electron- volt.

2. 	Which type of focusing is used in CRO?

3. 	Give the relationship between mobility and conductivity.

4. 	Define drift current.

5. 	If the PN junction is heavily doped the breakdown voltage will ____________.

6. 	Define forward current gain of CE transistor in terms of h parameter.

7. 	What is meant by latching?

8. 	Define negative resistance of tunnel diode.

9. 	What is photo conductive effect?

10. 	LED’s largely emit visible ____________.

PART – B  (5 x 3 = 15 MARKS)

11. 	Define electrostatic deflection sensitivity in CRT.

12. 	What is the potential barrier in PN Junction diode? For Silicon PN junction will the potential

barrier be about 0.7 V or 0.3 V?

13. 	What is Hall effect?

14. 	Draw the V-I characteristics of tunnel diode.

15. 	Distinguish between Photo diode and LED.

PART – C  (5 x 15 = 75 MARKS)

16. 	Obtain an expression for the motion of charged particle in the electrostatic field with initial velocity vxm/sec.

(OR)

17. 	With a neat sketch explain the function of CRO and also mention the applications of CRO.

18. 	Explain the Energy band diagram for open circuit condition of PN junction diode.

(OR)

19. 	Derive the continuity equations for holes and electrons.

20.  	Draw the small signal hybrid model of CE amplifier and derive the expression for its A i, A v, R iand Ro.

(OR)

21. 	Derive EberMoll equation from the basic current components in terms of equilibrium concentration of carriers in PNP region. Neglect recombination effect in base.

22.	Explain the working principle of a SCR and also explain its VI characteristics.

(OR)

23. 	Explain the symbols, constructional details, operation, Energy band diagram, V-I characteristics and applications of a Gunn Diode.

[P.T.O]

24. 	With a neat sketch explain the operation of LCD and applications and draw backs of LCD.

(OR)

25. 	Write short notes on

a.	Photo transistors             	b.	Nano crystal display                						(7+8)