Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ELECTRIC CIRCUIT ANALYSIS						      Time : 3 hours

Subject Code:	10EC202										       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In ________ circuit, input voltage is equal to the sum of the voltage drop across in individual resistances.

2.	Sign of E.M.F depends on ________ and ________.

3.	In RLC Series Circuit, if maximum current occurs at 25 Hz the resonance frequency fr will be _________.

4.	_________type of connection of three phase systems is preferred at the point of utilization.

5.	_________ Law is applicable for nodal analysis to find the node voltage in the circuit.

6.	Two coils of inductance 5H and 25H, then the maximum possible mutual inductance between those coils will be__________.

7.	The maximum power delivered if the RL is equal to __________.

8.	__________ voltage is equivalent value of Thevenin voltage.

9.	The transient occurs due to the presence of __________ elements in the circuit.

10.	The RL circuit consists of R=10Ω and L=10mH, the time constant will be_________.

PART – B  (5 x 3 = 15 MARKS)

11.	Convert the given Δ network  in the figure into Y network.

12.	A Series RLC Circuit is connected to a 220V, 50Hz supply when L is varied, the maximum current obtained is 0.4A and the voltage across the capacitor is 330 volts. Find the circuit constants.

13.	Find the “I” using current loop analysis method for the given circuit as shown in the figure.

[P.T.O.]

14.	Calculate the value of “RL” so that maximum power is transferred from the battery as shown in the figure.

15.	In the circuit shown in the figure, find the transient current when the switch is closed at t=0 and also initial rate of change of current. Assume zero initial conditions.

PART – C  (5 x 15 = 75 MARKS)

16.	Assuming the initial resistance of the batteries to be zero, calculate the current supplied by the two batteries in the network as shown in the figure.

(OR)

17.	a.	Find the currents in all the resistors in the circuits shown in the figure.

b.	Determine the current delivered by the same in the circuit shown in the figure.

18.	A Series RLC circuit has R=10Ω, L=0.54H and C=40μF the applied voltage is 100V. Find:

a.	Resonant Frequency 		b. 	Quality factor 	c. 	Upper and Lower half power frequencies 		d. 	Bandwidth vs. Current at resonance 	e. 	Current at half power points f.	Voltage across inductor.

(OR)

19.	a.	Derive the expression for total power in a 3- phase balanced circuit using Two-Watt meters     																				(8)

b.	The Power input to 200V, 50Hz 3- phase motor is measured by two watt meter which 	indicates 300KW and 100KW respectively. Calculate the input power, Power factor and 	line 	current.              															(7)

20.	Find the current through 2Ω resistor in the network using loop analysis and nodal analysis.

(OR)

21.	a.	 Find the Phasor voltage V2 in the circuit shown in the figure.				                 (10)

b.	Two identical coupled coil in series have an equivalent inductance value of 0.084H and 	0.0354H. Find the values of L, M and K.  								                   (5)

[P.T.O.]

22.	Find the voltage across 2Ω resistor shown in the figure by the principle of Superposition.

(OR)

23.	Determine the load resistance to receive maximum power from the source and find the maximum power delivered to the load in the circuit shown in the figure.

24.	Explain in detail about RLC transient for AC input.

(OR)

25.	A series RL circuit with R=100Ωand L=20H has a DC voltage of 200V applied through a switch at t=0. Find:

a.	the equation for current and voltage across different elements

b.	the current at t=0.5sec and 1sec

c.	time at which eR=eL.