Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title:	VLSI DESIGN											       Time: 3 hours

Subject Code:	10EC205								                     Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	In Wafer Formation, the atmosphere is typically helium or argon as to prevent the silicon from ________________.

2.	SOI has lower ________________ capacitance.

3.	Bird’s Peak effect mainly occurs in ________________.

4.	________________ is a good thermal insulator and an electrical insulator.

5.	Delays have no impact on ________________.

6.	What is ENTITY Declaration in VHDL?

7.	Metal to P-Active is called as ________________.

8.	Metal to N-Active is called as ________________.

9.	What is a VHDL Test Bench?

10.	What are the components in VHDL Test Bench?

PART – B (5 x 3 = 15 MARKS)

11.	What are the advantages of SOI?

12.	What are the Physical Limitations of Design Rules?

13.	Draw the Logic Diagram for x = (a + b). c

14.	Write a VHDL Program using CASE Statement for 4 to 1 MUX for the description given below.

| IO Pins | Description |
| --- | --- |
| a, b, c, d | Data Inputs |
| s[1:0] | MUX selector |
| o | Data Output |

15.	Write a VHDL Code for an 8 Bit Shift Register using PROCESS Statement with Positive Edge Clock, Serial Input and Serial Output for the PIN Description shown below.

| IO Pins | Description |
| --- | --- |
| C | Positive Edge clock |
| SI | Serial In |
| SO | Serial Out |

PART – C (5 x 15 = 75 MARKS)

16. 	Explain the P-Well on N-Substrate Process.

(OR)

17.	Explain Twin Tub and SOI Process.

18.	Draw the CMOS Circuits for the following.

a.	F=(a'+b')c				b.	OUT=D+A(B+C)

[P.T.O]

c.	Consider the following circuit built from NFETS and PFETS:

i.	Can this circuit be used as a CMOS gate? If not, explain why. If so, what function 		does it compute?

ii.	If we wanted the output voltage to change more quickly when going from logic "0" to

logic "1", what changes would we make to the FETS?

(OR)

19.	Explain Lambda Design Rules and Modes of Scaling in MOSFET.

20.	Two NMOS Inverters are cascaded to drive a Capacitive Load CL = 16Cg as shown in the figure. Calculate the Delay from Vin to Vout in terms of τ

(OR)

21.	Tabulate the various regions of operation of NMOS and PMOS in CMOS Inverter characteristics.

22.	Write the Behavioral code for the following diagram in VHDL.

(OR)

23.	Write the VHDL Code for 1: 8 Demultiplexer using Case Statement.

[P.T.O]

24.	Write the VHDL code for the Priority Encoder shown below.

(OR)

25.	Write the VHDL Test Bench for the diagram shown below.